9-21-04 Draft RP-65-P2

API RP-65 Part 2

"Isolating Potential Flow Zones in Well Drilling and Cementing Operations"

SPECIAL NOTES:

API publications necessarily address problems of a general nature. With respect to particular circumstances local state and federal laws and regulations should be reviewed.

API is not undertaking to meet the duties of employers, manufacturers, or suppliers to warn and properly train and equip their employees and others exposed, concerning health and safety risks and precautions, nor undertaking their obligations under local, state, or federal laws.

Information concerning safety risks and proper precautions with respect to particular materials and conditions should be obtained from the employer, the manufacturer or supplier of that material, or material safety data sheet.

Nothing contained in any API publication is to be construed as gaining any right, by implication or otherwise, for manufacture, sale, or use of any method, apparatus, or product covered by letters patent document. Neither should anything contained in this publication be construed as insuring anyone against liability for infringement of letters patent.

Generally, API standards are reviewed and revised, re affirmed, or withdrawn at least every five years. Sometimes a one-time extension of up to two years will be added to this review cycle. This publication will no longer be in effect five years after its publication date as an operative API standard or, where an extension has been granted, upon republication. Status of the publication can be ascertained from the API Upstream Segment [telephone (202) 682-8000]. A catalog of API publications and materials is published annually and updated quarterly by API, 1220 L Street, N.W., Washington, D.C. 20005.

This document was produced under API standardization procedures that ensure appropriate notification and participation in the developmental process and is designated as an API standard. Questions concerning the interpretation of the content of this standard or comments and questions concerning the procedures under which this standard should be directed, in writing, to the standardization manage, American Petroleum Institute, 1220 L Street, N.W., Washington, D.C. 20005. Requests for permission to reproduce or translate all or parts of the material published herein should also be addressed to the general manager.

API standards are published to facilitate the communication of proven, sound engineering and operating practices. These standards are not intended to obviate the need for applying sound engineering judgement regarding when and where these standards should be applied. The formulation, adoption, and publication of API standards are not intended to inhibit anyone from using any other practices.

Any manufacturer marking equipment or materials in conformance with the marking requirements of an API standard is solely responsible for complying with all the applicable requirements of that standard. API does not represent, warrant, or guarantee that such products do in fact conform to the applicable API standard.

FOREWARD:

Unless directed otherwise, the laboratory procedures in this document are performed according to API recommended practice on instruments that have been calibrated in accordance with the applicable standard.

This document was prepared with input from oil and gas operators, drilling contractors, service companies, consultants and regulators. It is based on experiences in the outer continental shelf and deepwater operating areas of the Gulf of Mexico, but may be of utility in other offshore and land operating areas.

API publications may be used by anyone desiring to do so. Every effort has been made by the Institute to assure the accuracy and reliability of the information contained in them; however, the Institute makes no representations, warranty, or guarantee in connection with this publication and hereby expressly disclaims any liability or responsibility for loss or damage resulting from its use or for the violation of any federal, state, or municipal regulation with which this publication may conflict.

Suggested revisions are invited and should be submitted to the standardization manager, American Petroleum Institute, 1220 L Street, N.W., Washington, D.C. 20005

Isolating Potential Flow Zones in Well Drilling and Cementing Operations

1.0 Scope

This document contains best drilling practices and primary cementing design recommendations for inhibiting annular fluid flow by barriers including hydrostatic pressure, mechanical seals, or cement.

1.1 Background

On August 16th of 2000, the MMS (Minerals Management Service) of the U. S. Department of the Interior presented safety concerns on uncontrolled annular flows to a new API Work Group comprised of government and industry representatives of several organizations including API Washington staff, API Executive Committee on Drilling and Production Operations, API Subcommittee 10 on Well Cements, International Standards Organization, International Association of Drilling Contractors, Drilling Engineering Association, the MMS, and other interested parties. Issues related to sustained casing-head pressure (SCP) were also discussed. The API Work Group on Annular Flow Prevention and Remediation agreed to prepare “best practices” to improve zonal isolation, reduce the occurrence of SCP, and help prevent annular flow incidents prior to, during, and after cementing operations. Studies of all available information on flow event causes and prevention helped the Work Group write best practice documents for publication as API Recommended Practices. The MMS is considering the incorporation of these API publications into new regulations for well construction in federally regulated areas. The API Work Group is responsible for the following API Recommended Practice publications:

- RP-65 Part One titled "Cementing Shallow Water Flow Zones in Deep Water Wells."

- RP-65 Part Two titled “Isolating Potential Flow Zones in Well Drilling and Cementing Operations.”

- RP-65 Part Three titled “Zone Isolation to Prevent and Remediate Sustained Casing Pressure.”

1.2 General

The failure of an annular cement column to isolate the wellbore is the root cause for many of the loss of well control incidents experienced in offshore drilling operations. Loss of well control events can occur at any point in the well construction process, from soon after spudding the well to drilling the well at total depth.

A number of factors are common to loss of well control events experienced while drilling the top-hole section. From a pressure maintenance standpoint, many wells are drilled in a near-balanced condition. As such there often are times when only a minimal pressure margin exists between formation pore pressures and circulating hydrostatic pressures of the drilling fluid. Typically the well is being drilled with simple spud muds with minimal fluid loss control. The cement slurries employed are lightweight, extended lead cement systems with a tail cement of higher density placed in the lower section of the cemented interval. In common practice, both lead and tail cement systems would be designed without any gas control capabilities. Further, certain lightweight cement systems are prone to gel quickly, thereby accelerating the loss of hydrostatic pressure exerted on the column of tail cement below.

It should be noted that in the Gulf of Mexico the majority of loss of control events occur after tophole cementing operations. Conventional cement designs typically employed in the shallow section of the wellbore possess no ability to mitigate flow, and as such, must not be used for the purpose of controlling annular flow in the wellbore.

Achieving zonal isolation in the presence of a potential annular flow requires not only the modification of the cement properties to facilitate control of migrating formation fluids but also several other actions including:

- A stable wellbore – no losses or gains

- Adequate annular circulating flow clearances

- Proper mud conditioning

- Spacer design

- Casing centralization

- Tripping requirements

- Drilling techniques

- Well monitoring

This document is a compilation of best-demonstrated practices, engineering considerations and cement property requirements to be applied to assist in the prevention of annular flows and to establish zonal isolation of the wellbore. This task entails, at its most fundamental level, the removal of mud from the wellbore and replacement of mud with cement capable of achieving and maintaining annular isolation.

1.3 Conditions of Applicability

The guidance from this document will cover two main areas: (1) Recommended drilling practices and cement design recommendations to mitigate annular fluid flow as a result of a pre-spud hazard assessment and (2), required actions to mitigate flow as a result of events experienced while the well is being drilled.

Pre-spud information gathered from offsetting well(s) and /or from high resolution seismic surveys can be used to define the degree of flow potential (Low, Medium, High) for a particular drilling prospect. Any relevant information should be communicated to the appropriate service provider for incorporation into the basis of design for a particular fluid (mud or cement) and for relevant engineering and operations procedures.

If any subsurface anomaly with a potential flow is identified by seismic or offsetting well information, the following recommendations are made:

- When possible, the surface location should be relocated outside the anomaly to a location with less risk.

- When relocation of the wellsite is not possible, the drilling plan for the effected section of the wellbore should be modified to mitigate the risk of flow. The next section on well planning details the recommended action steps.

2.0 Well Planning and Drilling Plan Considerations

2.1 Evaluate Well For Flow Potential

Before drilling a well, the operator should attempt to identify and analyze potential flow zones.  A variety of techniques are available to do this, three of which are discussed below.  The success of these techniques in identifying and successfully dealing with flow zones is related to the quality of the available data, a company’s experience in a specific geographical area, and the quality of personnel.

2.1.1 Site Selection

Prior to drilling, the operator can minimize encounters with potential flow zones by carefully selecting a site that achieves target depth while minimizing the risk of encountering a flow.  This is primarily accomplished through accurate review and analysis of available shallow and deep hazards data, proper interpretation of this information, and assimilation of this information into the drilling program.

When available, offset well information should be evaluated to determine if flow zones were encountered, the magnitude of any flow events, and the methods used to mitigate the effects of these flows.  If possible, this information should be incorporated into the current drilling program.

Cementing Shallow Water Flow Zones in Deep Water Wells, API Recommended Practice (RP) 65, First Edition, September, 2002, Section 4, addresses site selection for minimizing shallow flows in deep water wells. Many of the same principles discussed in this Section apply to operations for all water depths and well depths.  Refer to RP 65 for further information.

2.1.2 Shallow Hazards

Identification and evaluation of shallow hazards through the use of seismic surveys can aid the operator in proper site selection.  If available, shallow seismic data from offset wells or adjacent fields should be analyzed to verify the analysis.  If hazards are identified, they should be avoided if possible.  The operator is cautioned that shallow hazards analysis is not a conclusive method of prediction, so precautions to minimize the chances of a flow should still be implemented when drilling the shallow portions of the well.

If the decision is made to drill a well in an area that is likely to encounter shallow hazards, consideration should be given to drilling the shallow portions of the well with a small diameter pilot hole.  If a shallow flow zone is encountered during pilot hole drilling, a kill operation will be easier to achieve.

2.1.3 Deeper Hazards

Potential sources of deeper drilling hazards include abnormal pressure, drawn-down zones, faults, tectonics, salt flows, and areas of lost circulation.  Such hazards can often be identified through seismic interpretation and/or analysis of offset wells or fields.  Identification of hazards that could be encountered during drilling operations will aid in proper well planning and in minimizing risk.  If available, deep seismic data from offset wells or adjacent fields should also be analyzed to aid in the prediction of flow zones.

2.2 Planning the Well – introductory verbiage from DPP

2.2.1 Pore Pressure / Fracture Gradient / Mud Weight  – The well planner must understand the anticipated pore pressures, fracture gradients, and mud weights that will be encountered while drilling the well.  This information is generally presented as a graph, as shown in Figure 1.

INSERT FIGURE 1

General guidelines for the construction of this graph are given below:

- Plot the predicted pore pressure vs. depth, expressed as an equivalent mud weight (EMW).  It may also be helpful to note lithological information, if it is available.

- Similarly, plot the predicted fracture gradient profile.  Draw a design fracture gradient profile that is offset to the left of the predicted curve by a prescribed amount to roughly account for kick tolerance and the increased ECD during drilling and cementing operations.  Typical offset values range from 0.2 ppg to 0.5 ppg.

- Draw the mud weight profile based on the pore pressure and fracture gradient data.  In general, the mud weight profile is offset to the right of the pore pressure curve to provide a sufficient overbalance for trips.  Typical offset values range from 0.3 ppg to 0.5 ppg.

If mud weight and leak-off test (LOT) information is available from offset wells, include it on the graph for reference.  If large disparities exist between the offset well information and the predicted values, further investigation may be warranted. This graph will become the design basis for the well.

2.2.2 Casing Plan – The appropriate selection of shoe depths and consequently, the required number of strings is critical to the well design.  General guidelines are given here for the selection of shoe depths.  Local practices and experience should also be used to fine-tune this process.

Figure 2 - Casing Shoe Selection Graph

The initial shoe depth determination is made as follows (see Figure 2):

- Starting at the mud weight at the well TD (point A), draw a vertical line upwards until it intersects the design fracture gradient curve (point B).  This is the approximate shoe depth of an intermediate casing.

- Draw a horizontal line from point B leftwards until it intersects the mud weight curve (point C) and then upward until it intersects the design fracture gradient curve (point D).  This represents the approximate shoe depth of the next casing string.

- Repeat this process until all shoe depths dictated by mud weight and fracture gradient constraints have been established.

After the preliminary shoe depths have been established, an additional check should be made based on kick tolerance.  The kick tolerance is the maximum size kick of a specified intensity that can be circulated out of the hole without causing the formation to fracture in the open hole section (often near the shoe).  It may be necessary to adjust casing shoe depths to conform to kick tolerance limits.

In some higher pressure wells with a small margin between the mud weight and the fracture pressure, the recommended kick tolerance is nearly impossible to achieve.  This is particularly true for many wells drilled in the Gulf of Mexico.

There are numerous other factors that affect the design of shoe depths.  These factors include:

- Regulatory requirements.  Applicable local regulations should always be obtained before beginning the design.  For the Offshore U.S., MMS regulations can be found in CFR 30 part 250.

- Hole stability.  This can be a function of mud weight, deviation and stress at the wellbore wall, or it can be chemical.  Hole stability problems often exhibit time dependent behavior (making shoe selection a function of penetration rate).  The plastic flowing behavior of salt zones should also be considered.

- Differential sticking.  The probability of becoming differentially stuck increases with increasing differential pressure between the wellbore and formation, increasing permeability of the formation, and increasing fluid loss of the drilling fluid (i.e., thicker mud cake).

- Zonal isolation.  Shallow fresh water sands need to be isolated to prevent contamination.  Lost circulation zones should be isolated before a higher-pressure formation is penetrated.

- Directional drilling concerns.  A casing string is often run after an angle-building section has been drilled.  This avoids keyseating problems in the curved portion of the wellbore due to the increased normal force between the wall and the drill pipe.

- Uncertainty in predicted formation properties.  Exploration wells often require additional strings to compensate for the uncertainty in the pore pressure and fracture gradient predictions.

Hole and Pipe Diameters - The selection of pipe diameters has the largest impact on well costs in both design base and detailed casing design.  In general, hole and pipe diameters should be designed to be the smallest possible, which meet all design requirements, well objectives, and safety and environmental requirements.  In exploratory wells, hole diameters may be larger to allow for contingency casing string (s).  The final hole or casing diameter is generally determined by evaluation, completion, and production requirements.  Because of this, casing sizes should be determined from the inside outward.

Hole and casing diameters are based on the following requirements:

- Drilling - minimum bit diameter for adequate directional control and drilling performance, available downhole equipment, rig specifications, and available BOP equipment.

- Cementing – See next Section 2.2.3 below under Hole Size and Shape and Section 5.5 for more information.

- Production - production equipment requirements including tubing, subsurface safety valve, submersible pump and gas lift mandrel size, completion requirements (e.g., gravel packing), and weighing the benefits of increased tubing performance of larger tubing against the higher cost of larger casing over the life of the well.

- Evaluation - logging requirements and tool diameters.

2.2.3 Cementing Plan – Short and long term isolation of potential flow zones requires proper cementing planning and execution.  Several aspects of well planning may affect the success of primary cementing operations:

- Hole Size and Shape - Hole size and shape affects the available annular space, fluid velocities, displacement mechanics, centralizer effectiveness, and cement volumes.  Fluid systems and bottom hole assemblies (BHA) that provide a smooth, non-spiraling wellbore will improve displacement mechanics and ultimately cement placement. A non-spiraling hole shape can minimize casing eccentricity, improve uniform centralization by centralizers, and optimize mud displacement. In many wells, the available hole size coupled with needed casing size substantially restricts the annular flow area and virtually eliminates the ability to circulate cement in the well. The bypass flow area across tools such as liner hangers and liner packers should be maximized to help reduce flow restrictions and the associated high equivalent circulating densities (ECD) that can cause lost circulation, channeling, and poor cement placement. The ability to properly circulate the well with casing on bottom is key to successful placement. Section 5.5 has for more information.

  - Mud Conditioning - Conditioning of the mud before cementing will improve hole conditions.  Mud circulation will break gelled mud and place conditioned mud in the annulus. Care must be exercised to assure proper well control while conditioning the mud, and to assure that the well is stable (no mud losses or formation fluid influx or crossflows). If a well is flowing during cementing operations, there is a high risk of not getting a seal with the cement.

  - Spacers - Spacers are used for separation of the mud and cement and are also responsible for the bulk removal of mud from the well.  Spacers should be selected for compatibility with the mud and cement, and used in sufficient quantity to obtain the bulk removal of the mud.  Spacer volume should be based on the actual hole volume in the same manner that cement volumes are determined.  Well control calculations should include the hydrostatic pressure effect of the column of spacer in the annulus.

  - Pump Rates - Pump rate control affects ECD, mud displacement efficiencies, job time, and ultimately cement slurry design.  Consideration should be given not only to the displacement rate of the job, but to the mixing rate of the cement and its effect on the ECD and displacement efficiencies.  Cement mixing rates should not be designed so high as to prevent good density control.

- Centralization – In zones where an annular seal is critical, casing centralization can be a key to successful cementing.  Cement can not be placed completely around casing in places where the casing is laying against a formation.  Casing centralization will raise the casing off the formation and allow cement to completely surround the pipe.  Centralizer selection will depend on hole size/shape and casing size, and the directional profile of the well.  The use of a 3-D centralizer placement program that determines lateral loads on the centralizer, drag forces while running in the hole, and casing stand off is highly recommended.

- Testing/Evaluation Plan - Determining successful placement of cement can take many forms, from running of complex cement evaluation logs, temperature logs or simple pressure testing.  For most primary cementing operations, pressure testing is used to evaluate the effectiveness of the cement seal at the shoe of the casing.  Well planning should take into account how this test will be performed, and what type of testing is necessary. The plan should state whether the casing shoe test will be taken to leak off, or if the test will be stopped at a predetermined pressure.  Depending on drilling requirements, it may be appropriate to evaluate the need for open hole leak off or integrity tests. These tests can be run after reaching the planned hole section depth to confirm that the open hole’s pressure containment integrity will hold the designed maximum cementing job ECD. If the test indicates a lack of adequate integrity, the cementing job design may be modified accordingly. For example in some cases, a lower cement slurry density may be designed to lower the cementing ECD.  More information on testing and evaluation is in Sections 6 and 7.

2.2.4 Drilling Fluids Plan  – missing most of this section

2.2.4.1 Mud Weight vs. Pore Presure

Missing

2.2.4.2 Mud Stability

Missing Drilling Fluid Stability (settling, sag, etc.)

Hydrate Prevention - Gas hydrates are simple crystalline chemical compounds composed of water molecules surrounding a single free-floating natural gas (methane) molecule.  The crystal is arranged in a compressed latticework, appearing very similar to ice formed from water.  Unlike ice, however, hydrates can form at temperatures above the freezing point when system pressures are elevated.  Hydrate formation has led to plugging of wellbores, flowlines, and wellhead and well control equipment, and has jeopardized operators’ abilities to maintain positive control of pressures in the wellbore.  The prevention of gas hydrates is thus a critical concern in all deepwater wells.

Pre-emptive measures should be taken to insure that hydrates do not form and do not become a safety/well control problem.  The key to hydrate prevention is to inhibit the formation of hydrates through the proper selection and design of the drilling/completion fluid.  Low dosage gas hydrate inhibitors (anti-agglomerants) and a combination of anti-agglomerants/kinetic polymeric inhibitors have been successfully tested for the prevention of hydrate formation.  Other methods including the use of solvents such as methanol, maintaining the proper drilling fluid salinity and the use of non-aqueous base fluids can also reduce the likelihood of hydrate formation.  Computer models that predict the formation of hydrates under various conditions are available to the operator from a variety of sources.

Combine above with section below?

### Gas Hydrates

Gas hydrates can cause severe well control problems by forming a plug in valves or chokes completely blocking flow.  Upstream pressure then increases which compounds the problem.  For this reason, a frequent practice is to suppress the formation of hydrates by injecting inhibitors into the drilling fluid.  On the other hand, in many situations it might be desirable to avoid dissociating hydrates occurring in-situ.  Recent experiments have shown that these conflicting requirements can be achieved by using a special mud additive known as polyvinylcaprolactam (PVCap) which is capable of suppressing both hydrate formation and dissociation.

Hydrates are easier to prevent than to remove once they have formed in a flow system.  Prevention measures include

- good well control practices to minimize gas coming from the formation

- reduce free water by using oil-based mud or maximizing the chloride content of a water-based mud.

- maintaining the temperature as high as possible

- injecting methanol or glycol at a rate of 0.5-1 gallon per minute on the upstream side of a choke or line

If hydrates do form and plug off a choke or line consider circulating hot brine solution.

2.2.4.3 Mud Type

Missing section.

Also see Section 5.3 for more information on drilling fluid type.

2.2.4.4 Packer Fluid Stability – Packer fluids are fluids that are left in the annulus of the wellbore when the well is completed and put on production or temporarily abandoned.  The purpose of a packer fluid is to provide hydrostatic pressure control in the tubing/casing annulus and provide for corrosion protection of the tubulars in the well.

A properly designed and placed packer fluid should provide:

- Suspension - Suspension of weighting material such as barite, hematite or illmenite to prevent solids settling around packers or downhole equipment.  Settling of weight material can also lead to an unbalanced fluid column, which can lead to well control problems.

- Corrosion protection - Protection of downhole tubulars from corrosion is essential.  Pretreatment of the packer fluid with scavengers and film-forming compounds is essential for brines and water based fluids.  Non-aqueous (NAF) packer fluids normally provide adequate corrosion protection if the fluid alkalinity is in the desired range when the fluid is placed.

- Pump-ability - The fluid should be fluid enough to be pumped into place, yet viscous enough to provide the desired suspension of weighting agents. Properly conditioned packer fluids should be shear thinning so as to allow proper placement without exceeding the desire pumping/placement pressure.

2.2.5 Hydraulics - ECD

2.2.5.1 Equivalent Circulating Density (ECD) Management

A plan should be developed to monitor and control static and dynamic fluid pressures of the mud column such that ECD is maintained within appropriate limits.  Static fluid pressure should be sufficient to contain maximum openhole formation pressure and minimize wellbore stability problems, while dynamic fluid pressure should be held below the minimum breakdown (fracturing) pressure of any exposed formation.  Hole size, casing size, BHA and drill pipe size selection must be balanced with the fluid properties and surface equipment ratings to ensure ECD can be maintained within the desired range.  Offset well files should be reviewed for indications of lost circulation, stuck pipe, significant borehole washout, etc., and the ECD management plan should be modified to mitigate these problems.  ECD increase in high-angle and horizontal wellbore sections must be addressed in the plan, as the formation fracture gradient will remain constant in the horizontal section of the well.

Numerous well design parameters can impact ECD, including the use of casing strings with increased annular clearances, use of liners rather than full casing strings, selecting fluids that reduce frictional losses, expandable tubulars to preserve hole size, and controlling ROP to avoid overloading the annulus with cuttings.  Critical circulating and swab pressure should be documented in the plan.  A wellbore hydraulics simulator should be used on each well.

2.2.5.2 Wellbore Cleaning

Cuttings transport to the surface is primarily controlled by annular velocity and fluid rheology, so care must be taken when selecting components that impact these parameters.  Surface drilling equipment (mud pumps, flow lines, and shakers) must be sized to accommodate the maximum rate of cuttings generated.  Hole deviation must be considered when designing higher annular velocities for proper hole cleaning.  Ensure chemical compatibility of the formation and the mud system to avoid swelling problems.  The bit nozzle selection should be based on optimizing ROP, bit cleaning, and annular velocity for transport of solids to the surface.  It is recommended that a wellbore hydraulics/hole cleaning simulation model be run on each well to determine minimum and maximum flow rates.  Maximum pipe-tripping speed should be controlled to avoid creation of excess swab pressure.  Wells drilled from floating vessels should consider the use of a booster line when high solids loading is expected in the riser.

Conventional hole cleaning tactics may have to be supplemented with hole cleaning pills under certain conditions.  Viscous pills and high weight pills can be effective at removing cuttings.  However, the use of pills should be minimized where possible, as they can contaminate the mud system.

2.2.6 Well Center Planning

The well center design (i.e. wellhead, BOPE, riser, etc.) should include a minimum of two barriers, available during any wellbore operation, to prevent uncontrolled flow from the well to the atmosphere.  The barrier design will incorporate the following elements:

- Ability to withstand the maximum anticipated wellbore pressure

- Ability to be tested (leak & functional)

- Failure of a single barrier will not result in uncontrolled flow from the well

- Field operating environment is within the design specifications of the barrier element

In addition, one of the barriers should have the capability to:

- Shear any device that passes through the barrier and seal the wellbore after having sheared the tool.  If this is not possible, an alternative pressure control plan shall be created.

- Seal the wellbore with any size device that penetrates the well barrier.  If this is not possible, an alternative pressure control plan shall be created.

2.2.7 Barrier Plan

The ultimate goal of any well is to provide sufficient barriers between formations and between those formations and the surface.  A barrier plan includes maintaining well control with fluids that exceed pore pressure in the well, selection and use of well control equipment, and ultimately the placement of cement or other mechanical barriers in the well.  Evaluation of these barriers must be considered in the planning process.  Plans must address well control issues each time a barrier is removed or replaced.  Following a cement job, the BOP system is typically removed to install wellhead components.  At these times, the barriers in the well have changed from fluids and the BOP stack to fluids and cement.  The plan should address when it is safe to make that change.

Plans for testing of well barriers should be part of each well design.  Barrier plans should address pressure integrity through pressure testing, but may also require negative testing of a liner top prior to changing out the wellbore fluid.

See relevant Sections below for more information on designing the different types of barriers.

2.2.8 Contingency Planning - [Section “h” still being edited]

Unintended transfers of fluids or solids between the wellbore and the formation can occur with minimal warning and can have devastating effects.  The potential for such problems should be identified in pre-drill analyses.  Contingency plans should be developed specifying the procedures, equipment, and personnel needed to avoid adverse situations or to suppress incipient dangers before they become unmanageable.  Planning should consider events that fall into three categories: fluid influxes, lost circulation, and formation failure.

## 2.2.8.1 Well Control Planning for Fluid Influxes

## Kicks - The following equipment and materials should be available to handle a kick:

- A minimum of 300 barrels of heavy mud must be ready in the reserve pit (mud weighting 2 ppg more than the active mud is normally recommended).  However caution must be exercised to ensure that this mudweight will not lead to rupturing of the formation

- A diverter where shallow gas hazards exist

- Properly selected and well maintained blowout preventers, chokes, choke manifolds, mud-gas separators, degassers, mud monitoring equipment and other well control related equipment

Safety and well control procedures must be implemented if a kick occurs.

Well control procedures vary depending upon whether the kick occurs while tripping, drilling, or the bit is out of the hole, and whether surface or subsea BOP's are employed.  However in general they include:

- The use of kill mud

- Diverting shallow gas

- Temporarily shutting in the well

- Plugging the borehole with barite or cement plugs leading to partial or complete abandonment of the well

If the well is shut in, use a well control method that keeps the bottom hole pressure constant while allowing the kick to migrate through the flow system.

## Shallow Water Flows - Shallow water flows are best managed by drilling the interval with weighted fluid.  For cementing, foamed cement is recommended since unlike conventional cement, it continues to transmit the mud pressure throughout the thickening process. More information on controlling shallow water flows can be found in API RP-65 Part 1 titled “Cementing Shallow Water Flow Zones in Deep Water Wells” published in September 2002.

## 2.2.8.2 Planning for Lost Circulation Control

## There are generally three categories of losses: seepage losses, partial losses, and complete losses.  The two most common (and most avoidable) causes of lost circulation are excessive downhole pressures and improper selection of intermediate (protection) casing seats.  Rigsite operations should be directed toward avoiding wellbore pressures that can break down the hole. Downhole pressure can be divided into three types: hydrostatic, frictional (circulating), and inertial (surge/swab).

Hydrostatic pressure is controlled by using the minimum reasonable mud density, by maintaining adequate hole cleaning, and by controlling ROP as required.  Drilling with the minimum reasonable mud density is obviously the first consideration. Reducing the mud density not only lowers the hydrostatic pressure, it also lowers the surge/swab pressures and usually the rheological properties that affect circulating pressures. Other recommendations include the following:

- Avoid surging the pumps, avoid sudden starts, and break circulation cautiously after engaging the rotary while slowly pulling up on the drillstring. Engaging the rotary before breaking circulation eliminates the effects of downhole gelation. Slowly pulling up on the drillstring while breaking circulation offsets the inertial surge pressure.

- Avoid sudden starts and stops of the drillstring and control maximum pipe running speeds, especially in deep and small-diameter holes.

- Try drilling, not spudding, all bridges.

- During casing and cementing operations, care should be exercised to ensure that surge effects do not cause mud losses.

If losses occur, use the correct blend of loss control materials (LCM) depending upon mud type and the severity of the problem.  It is important to understand the relationship of particle sizes required to plug various fracture widths and to ascertain whether the loss is into formation matrix or a fracture. In most cases, continuing lost-circulation problems exist as a result of fractures.

In cases of losses to large vugs or caverns, about all that can be done is to continue drilling ahead without returns to surface (drilling blind). In some cases it may be desirable to pump large volumes of water down the annulus while drilling ahead blind. Sometimes it is possible to fill the voids with drilled cuttings to regain circulation if the void is small enough, but this is usually not the case. In most cases, the recommended practice is to drill blind to some point below the loss zone where a satisfactory casing seat can be attained. At that point, run casing and cement to the loss zone to seal it off.

Wellbore breathing or ballooning is often an indication of imminent lost circulation.  The following responses should be adopted:

- Review system hydraulics in an effort to reduce ECD.  Consider reducing the mudweight if possible

- Monitor flowback while making connections.  Record the volume and duration of flow.

- Do not weight up and risk loss of returns

- Apply an LCM pill

- When a kick and breathing both occur it is time to set casing

## 2.2.8.3 Formation Failure

## Formation failure caused by mechanical or chemical instability results in hole enlargement, an influx of cavings into the wellbore, and potentially severe consequences for drilling, logging, and cementing operations.  Procedures for avoiding or managing formation failure vary depending upon the type of formation, the nature of the instability, and the availability of time and resources.  However in general the following practices should be observed:

- Maintain the ECD within planned boundaries

- Use an appropriate mud type that promotes the formation of filtercake, cuttings transport, and inhibits chemical reactions with the formation

- Avoid surging or swabbing the formation and reduce tripping speeds when the BHA is opposite problem formations

- Plan frequent reaming or wiper trips in mobile formations

- Minimize the exposure time in areas where rock deformation or failure is time dependent (e.g. in salt)

- Perform adequate circulation to clean the hole before drilling ahead

- Drill problem formations with smaller hole sizes as this reduces hole cleaning problems and increases the mechanical stability of hole.  Alternatively use a larger drillpipe to improve cuttings transport

- Use reciprocation and rotation of the drillstring to assist hole cleaning particularly when mud flow alone is unable to clean the wellbore

- ROP should be controlled in upper section to avoid overloading the annulus

Good hole cleaning is especially important in deviated wells where cuttings beds can accumulate and avalanche down the low side of the wellbore packing off the drillstring.  In the event of a packoff, the following measures should be applied in order of appearance until the drillstring is freed:

- Turn off the pumps and bleed down the standpipe pressure

- Apply maximum make-up torque

- Working the pipe up and down

- Increase the standpipe pressure and continue to apply torque and work the pipe

- As a last resort, commence jarring in the direction opposite to the last pipe movement

2.2.9  Regulatory Issues

One component of proper well planning includes appropriate regulatory review.  Typically, the regulatory agency with jurisdiction for the well will need to review the well plan before operations begin.  Well plan submittal documents tend to have the following features in common: a description of drilling objectives, planned casing and cementing program, drilling fluid program, equipment testing policies and procedures, borehole evaluation and directional survey programs, and estimates of pore pressures and fracture gradient.  Depending on the type of regulatory system in effect, the agency may or may not approve the well plan in its entirety, or it may require certain changes to the plan to meet regulatory requirements.

2.2.10  Communications Plan

A key feature of good well design is effective communication of the plan to the personnel that will execute that plan.  The well design should be communicated through a written plan and through personnel meetings.  For complicated wells, holding a meeting to "drill the well on paper" can often highlight areas of risk and concern, and gives an opportunity for all parties to understand their role in executing the well program.  A discussion of human factors, required training and experience of personnel should be highlighted.

Pre-spud meetings should take the final well plan and highlight any areas of concern.  The meeting should review the full well plan in detail, highlight and review with the personnel the need for safety, and address topics related to annular flow prevention.  The prevention of annular flow does not rest solely with the cementing operation and well control equipment.  It is a process that involves wellhead equipment, directional control, wellbore quality, drilling fluid management and cementing operations.

3.0 Drilling the Well

3.1 General Practices While Drilling

The following practices should help maintain efficient drilling results and provide hole quality conditions suitable for primary cementing:

- Using a drilling fluid of sufficient density to contain formation fluids

- Use of high viscosity sweeps to reduce potential for annular pack off or excessive gumbo deposition

- Control mud losses using LCM as needed

- Minimize static gelled mud with flat gel strength mud rheology

- Prevent excessive mud cake buildup with low fluid loss muds

- Prevent balling on the BHA due to gumbo

- Rotate pipe while breaking circulation to reduce lost circulation potential on connections

- While on diverter, the rig should pump out of the open hole and assess trip fill by monitoring each stand while circulating through the trip tank

- Control ROP to prevent overloading the wellbore with cuttings and minimizing the opportunity for gumbo accumulation

- Monitor mud gas content and volume to verify flow potential.

3.2  Monitoring and Maintaining Wellbore Stability

Having a stable wellbore prior to, during, and after the cement job execution is crucial to cement job success.  If losses or gains occur during a cementing operation the possibility of obtaining a successful cement job is greatly diminished.  Corrective action should be taken to stabilize wellbores prior to the cementing operation. Certain corrective measures are best applied prior to running the casing or liner string into the well. The following indicators are used to identify potential flows and losses, contributing to wellbore instability:

- Changes in pit volume – monitor trend

- Changes in flow rate – monitor trend

- Changes in Pump Pressure

- ROP – monitor trend

- Torque and Drag

- Changes in WOB (weight on bit) – monitor trend

- PWD (pressure while drilling) data

- ECD

- Fracture Gradient via shoe and open hole LOT data

- Cuttings size and shape

- Variation in “d” and “dxc” exponent

- Monitor Gas – background, connection, and trip

- Flow-line temperature

- Mud properties; look for mud cut density, change in salinity, oil in retort, etc.

- Shale density

- Formation changes from LWD/MWD data

- Presence of geologic anomalies

  - Fractures

  - Faults

  - Non-conformities

Many of these indicators are precursors to wellbore losses or kicks which can result in an unstable wellbore.

3.3  Curing and Preventing Lost Circulation

Detailed well planning and accurate hydraulic modelling is extremely important in minimizing lost circulation. Lost circulation is one of the most common and expensive problems that are encountered while drilling or while running casing.  Loss of circulation can lead to loss of well control and a multitude of associated problems. The financial consideration is of concern with any type of drilling fluid but the importance is the greatest when using non-aqueous fluids (NAF).  Loss of circulation can lead to well bore instability and well control problems that can drastically affect the outcome of drilling and cementing of the well.  Loss of circulation occurs when either of the following conditions are met:

- The pressure exerted by the mud column either while static or during circulation (surge is considered as circulating) must exceed the formation fracture or pore pressure.

- The porosity and permeability of the formation or the fissure or fracture is large enough to permit the passage of whole mud thus preventing the sealing effect of the filter cake.

Loss zones can be categorized into two types: natural and induced losses.

3.2.1 Natural Losses

- Loss through rock permeability – for whole mud to be lost, the formation openings are larger than the largest particles contained in the drilling fluid.  These types of formations are usually characterized by seepage losses occurring in highly porous formations which are usually encountered at shallow depths.  Typically these formations are sands and gravel.

- Formations containing natural fractures.

- Cavernous and vugular formations – formations such as limestone or dolomites that have been dissolved by ground water that can lead to vugs or caverns.

3.2.2 Induced Losses

Losses due to a mechanical disturbance of the well bore can create fractures not normally present in the formation.  The hydrostatic pressure thus created in the well bore exceeds the formation break down pressure and mechanically fractures the rock.  These types of losses are different than what is seen in natural fractures since the fracture network is not interconnected.  Some of the causes of induced fractures are affected by the following:

- Fluid density and/or equivalent circulating density (ECD).

- Additional hydrostatic due to length of riser.

- Insufficient hole cleaning.

- Rate of penetration (ROP)

- Pump rates

- Drilling fluid rheological properties

- Movement speed of the drill string or casing.

- Wellbore geometry

- Restricted annulus from pack offs or bottom hole assembly (BHA) balling.

3.2.3 Loss Rate Categories:

Losses of mud to the formation have been arbitrarily defined in the following categories:

- Seepage losses         =	1 - 20 bbl/hr.

- Partial Losses           =	20 - 50 bbl/hr.

- Severe Losses          =	>50 bbl/hr but the hole will remain full with the pumps off.

- Complete losses       =	No returns while pumping or the hole will not remain full

with the pumps off.

#### 3.2.4 Preventing Losses

# Drilling Fluid Properties

The following should be done to optimize the drilling fluid for preventing losses:

- Proper solids control management.

- Keep the fluid density as low as possible.

- Maintain gel strengths and yield point at the lowest levels that will effectively clean the hole and effectively suspend barite and cuttings.

- Prevent excessive filter cake build-up by controlling fluid loss.

- Utilize hydraulic prediction software to predict ECD and determine optimum fluid properties.

Minimizing Surge Pressures

Surge pressures can be minimized by:

- Staging in the hole to prevent excessive circulating pressures.

- Rotating the pipe before turning on the pumps and bringing the pumps up slowly.

- Monitor and control pipe running speeds.

- Utilize available hydraulics modeling software for predicting surge pressure.

## Downhole Equipment

The following downhole equipment practices will help reduce ECD to limit losses:

- Utilize down hole pressure measurements to monitor and manage ECD’s in real time

- Utilize tools with maximum annular flow paths across the tools.

- Auto fill and various enhanced flow by-pass equipment could be used to minimize surges while running casing.

- Use down hole tools that allow for high concentrations of loss circulation material (LCM).

3.2.5  Identifying the Loss Zone

Quickly identifying where the loss zone is located will greatly enhance the performance of the treatment used to counter the lost circulation.  Temperature logs, spinner surveys, noise logs, logging while drilling (LWD) data analysis and stress modeling, connection flow monitoring analysis, lost circulation computer model simulations, and offset well mud loss data  are a few of the techniques used to identify where the suspected loss zone may be located.  If a NAF is used, the suspected loss zone could be identified during a short trip by very high resistivity readings in an otherwise non-productive zone.

3.2.6  Lost Circulation Materials and Systems

There are a wide variety of lost circulation materials available to deal with the most severe types of lost circulation.  There is no generic treatment available that is universal to all types of lost circulation encountered.  These can range from fine to coarse particulate materials, fibers, cements, reactant pills and acid soluble particulates. The type of material to use should be matched with the severity of the lost circulation encountered.

Seepage Losses

The most common type of lost circulation materials for this type of losses are the fine cellulosic fibers and fine granular types of additives.  The most commonly used granular material is calcium carbonate.  Another widely used technique is to pump the fine seepage loss additives as a sweep while drilling the seepage loss zone.

Partial Losses

The most commonly used materials for this type of loss circulation are granular and flake types of products that have particles sizes larger than those used to deal with seepage losses.  These can include mica, nut shells, and medium to coarse calcium carbonates, graphitic materials and coarse cellulosic fibers.  Special graphitic products are also effective in dealing with these types of losses.  Generally, the graphitic type of additives has a particle size larger than the cellulosic fibers and fine granular type (calcium carbonate).  These additives can be added to the mud system as a continuous treatment or can be spotted as a sweep across the suspected loss zone.

Severe Losses

The most commonly used materials for severe losses are the granular and flake type products that have larger particle sizes than those used to deal with partial losses.  As the severity of the loss is greater than the preceding two types of losses, coarser types of the same additives should be used as well as including some of the fine sized materials.  Special graphitic products are also effective in dealing with these types of losses.  Generally, the graphitic type of additives has a particle size larger than the cellulosic fibers and fine granular type (calcium carbonate).  These additives can be added to the mud system as a continuous treatment or can be spotted as a sweep across the suspected loss zone.

Complete or Total Losses

The types of additives used to deal with total losses are generally a different class than is required for the preceding types of lost circulation.  These types of losses generally occur in formations with leak off flow paths larger than the diameter of some types of bridging particles. Large fracture openings and vugs usually account for these massive losses of drilling fluid. These openings are not effectively sealed with the cellulosic and granular type of products described above. Reactive pills and agglomerating type LCM materials are used under these circumstances.  Some of these are as follows:

- High filtration squeezes allowing for rapid loss of the carrier fluid resulting in a solid plug forming in the formation opening.

- Hydration type systems where a very active material such as powdered clay or mixtures of clays and polymers in an inert carrier reacts with the drilling fluid to form a rubbery type of plug.

- Chemical type systems where special resins or polymers and catalysts react to form a semi-solid type of plug can be used.

- Mixtures of the above systems with cement to create highly viscous and hard setting plugs may also work.

- These types of losses also respond well to cement plugs to cure the lost circulation.

- Large cavern loss zones may need large volumes of the above systems or special squeeze cements such as foam cement.

3.2.7  Planning and Operations Considerations

- Pre-well planning for all phases (drilling, running casing/liners, & cementing)

  - Drilling

    - Pore pressures and fracture gradients

    - Fluid selection – NAF vs water based mud (WBM)

    - Minimizing flow properties

    - ECD control, annular pressure measurements

    - Controlled ROP and hole cleaning

    - Caliper logs

    - Lost circulation, stress cage, and ECD computer modeling

  - Running casing and liners

    - Swab and surge pressure modeling

    - Insure the hole is clean and free of cuttings

    - Casing hardware selection

    - Pre-treat the drilling fluids to reduce pressures

    - Breaking circulation

  - Cementing

    - ECD modeling

    - Utilize good cementing practices

    - LCM in the cement slurries

    - Spacer considerations – flushes, Spacer design (hydraulics)

    - Use proper cement densities and cement volumes.

- Pre-treating the mud

  - Drilling

    - Pre-treat with various particle size distribution (PSD) materials

  - Running casing

    - If possible cure the lost circulation prior to running casing.

    - Spot LCM in the open hole prior to running casing

  - Lost circulation and stress cage computer modeling

4.0  Mechanical Barriers and Operational Considerations

4.1  Introduction and Flow Incident Cases

Objective

The objective of this guideline is two fold. The first is to prevent and / or control flow after cementing on casing strings through surface pipe. These incidents of loss of well control are a very serious problem threatens the safety of offshore personnel, the rigs themselves, and pollution of the environment. The second objective is the elimination of sustained casing pressure (SCP) which is also a serious industry problem.

Background

The data presented primarily by the MMS obtained from industry, strongly suggest that the “Incidents of loss of well control”, particularly when cementing or after cementing shallow strings, is a serious problem and blemish to the drilling industry. Events like this are a threat to human life, a source of pollution, expensive, and affect the industries ability to obtain leases in some areas because we are considered unsafe, and the potential for a pollution event like those presented is a real concern.  Since anything and everything can happen on a cement job, even when it has been properly designed, other means of preventing these “incidents of loss of well control” need to be employed when there is a risk of hydrocarbon flow involved.

MMS regulation 30 CFR 250.422 (b) requires that if you plan to nipple down the diverter or BOP stack during the 8 hour or 12 hour WOC time you must determine before nippling down when it will be safe to do so. Your decision must be based on your knowledge of the formation, cement composition, effects of nippling down, potential drilling hazards, well conditions, and past experience. Even though we have this regulation, more specific guidance is needed since we are still having well control incidents. This is unacceptable and it effects out entire industry.

Flow Incident Examples from MMS Records.

The following are examples taken from actual situations to emphasize the potential consequences of annular flow events that took place during or after cementing operations on OCS (outer continental shelf). Some of the similarities that reoccur most often are:

- Most events took place during or after cementing of surface casing.

- Most wells used a mudline hanger / suspension system.

- Frequently the surface x conductor annulus was washed out after cementing.

- Washing out of the annulus resulted in a small but possibly very significant changes in hydrostatic pressure.

Flow Incident Example 1.

| Safety Alert No. 199 November 15, 2001 | Contact: Buddy Stewart 337-289-5108 |
| --- | --- |

Blowout and Fire

After a slip-on wellhead (SOW) had been set on a 10¾-inch surface casing, and during the welding of the inside of the SOW, gas and liquid were observed escaping from the +10 valve. The +10 valve was closed and 8.6-ppg seawater was added to the annulus through a ½-inch gap between the wellhead’s 24-inch base plate and the 10¾-inch casing, in an attempt to kill the well. It should be noted that a 16-inch scab liner was hung from the drive pipe. The well continued to flow, so the fluid was weighted up to 9.2-ppg mud, and then later increased to 16-ppg mud. It became more difficult to maintain a steady rate and quantity of mud flow because of the mud’s density and the size of the opening between the base plate and the surface casing. As the gas flow increased, the +10 valve was opened in order to allow the 16-ppg mud to displace the lighter fluid in the 24-inch x 10¾-inch annulus. With gas flow still increasing, the decision was made to evacuate the rig. The +10 valve was left opened in an effort to divert the flow of gas away from the platform and rig. Approximately a day later, fire erupted on the platform from an unknown source and caused extensive damage to the platform and drilling rig.

The MMS investigation of the accident revealed in part that, while the source of the blowout, namely shallow gas, was identified through well log data, the cement program was not properly designed for the presence of shallow gas. The failure of the operator to communicate the presence of shallow gas to the contract cement service company resulted in the improper design of the cement. It was further revealed that a pre-spud meeting, in which the presence of known shallow gas could have been communicated to all parties involved in the drilling operation, was not conducted.

It is therefore recommended that operators:

- Review their current specific strategies regarding the prevention of gas migration into the cement column.

- Review their method of communication with contract cement companies regarding shallow-gas hazards.

- Review their procedures by which well log information indicating shallow gas is used in the proper design of their cement program.

Flow Incident Example 2

Notice No. 165

April 3, 1995

Shallow Gas Flows While Cementing Surface Casing

A well recently blew out while waiting on cement after surface casing was set. After the wiper plug was bumped, the casing was pressure-tested and sugar water was spotted on top of the mud line hanger. The well was static for four hours, and the operator commenced to nipple down the diverter system. Two hours later, the well started flowing gas and cement out the open diverter. The diverter was closed, and the loosened bolts that attach the diverter to the conductor casinghead were reinstalled. Efforts were unsuccessful to stem the flow by pumping saltwater through the conductor casing valve while allowing the well to unload out both 10-inch diverter lines. Approximately one-half hour after the well started unloading, flow began coming from the annulus between the drive pipe and the conductor, and the rig was evacuated.

Several unsuccessful attempts were made to control the flow. Six days after the flow began, the flow rate decreased and the rig was able to move off location. Four days later, the drive pipe, conductor, and surface casings fell to the seafloor and the flow ceased. The next day, a subsea inspection by a remote operated vehicle (ROV) revealed a 25-foot crater at the base of the drive pipe with no bubbles or flow observed. Five months later, another rig moved on location and successfully completed abandonment of the well. There were no injuries or fatalities as a result of this blowout.

A similar incident occurred in July 1994 while spotting water mixed with lignosulfonate (ligno water) after washing the annulus through wash ports on the mud line hanger. After the 20-inch surface casing was cemented, the annulus was washed by circulating through ports on the mud line hanger, and ligno water was spotted. After a gyro survey was run, a diverter line was opened to check for flow. Flow was noticed on the annulus and the diverter line was closed. The annulus pressure was alternately bled off and allowed to build up during the next day. During the next two days, unsuccessful attempts were made to place heavy mud into the annulus by injection and by circulating through the mud line hanger. The well was finally killed by placing 3/4-inch tubing into the annulus and circulating heavy mud. Operations were resumed some three days after the flow began. There were no injuries or fatalities associated with this incident.

In another, less serious incident, a shallow gas flow also occurred after the surface casing was cemented. Nippling down of the diverter was begun seven hours after the plug was bumped and the float valves were checked. The diverter and diverter lines were rapidly reinstalled when flow first began. The well was shut in with a surface pressure of 50 psi, and was killed by lubricating mud into the annulus between the conductor and the surface casings. The annulus was then successfully grouted, and normal drilling operations were resumed 5 3/4 days after the flow first began. There were no injuries or fatalities as a result of this incident.

In still another incident, nipple down of the diverter system and installation of the casing slips were completed two hours after the plug on the surface casing was bumped. A slight gas flow was ignited by a welder making a rough cut on the surface casing one-half hour later. The gas was flowing from a casing valve that was open so that the annulus could be monitored. The flame was extinguished with no damage or injuries when the casing valve was closed. However, when the valve was later reopened, mud and gas flowed from the well. The valve was closed and the diverter was reinstalled on the wellhead. At this point, the seal on the casing slips failed, and 15-20 bbls of mud and gas were discharged through the diverter line. When fluid slowed to a very small stream, a line was connected to the casing valve, and 3 3/4 bbls of mud was pumped between the 13 3/8-inch and 20-inch casings. The well was monitored for 16 hours and 6.9 bbls of mud were recovered. Final flow rate was 2 1/2 gallons per hour, and at this point, the crew prepared to resume normal operations. The lead cement had thixotropic properties typical of slurries used for this application.

All four cases were caused by formation fluids migrating into the annulus as the cement went through a transition before compressive strength was developed. Sixteen similar well-control incidents have been reported since 1973.

The best way to avoid these problems is to select, on the basis of the shallow hazards survey, a surface location that is not directly above a seismically visible shallow gas accumulation. However, if well objectives require drilling from a location in a shallow gas area, or if it is likely that gas sands will be penetrated in the surface hole, appropriate consideration as to how to handle any shallow gas related problem that might arise should be developed when the well is planned. This consideration is particularly important where the hydrostatic head is to be reduced by activities such as washing mud or cement from the upper annulus with water to facilitate future abandonment.

Items to be considered in shallow gas areas may include the following:

1. Improving cement properties to minimize chances of shallow gas flow, including density, fluid loss, transition time, compressibility, etc.

2. Planning for optimum cement column length (within guidelines established in 30 CFR 250.54).

3. It may be desirable to hold slight back pressure on the annulus after cement is in place, and to consider shut-in of the well rather than diverting the well. In these cases, leakoff testing of the conductor casing shoe is recommended to better predict the surface pressures that could later be withstood without fracturing. Based on results of the leakoff test, the amount of back pressure that could safely be applied until cement has achieved compressive strength should be calculated. Any applied back pressure should not be enough to cause the formation at the conductor shoe to break down, and the annulus should be bled as required to avoid breakdown.

The considerations made regarding shallow gas should also seriously address whether or not the parameters associated with a particular well suggest diversion after cement is in place as a viable option. These parameters include leakoff test data and various depths at which gas sands may be drilled in the surface hole. The mindset that "diversion is the best option because a diverter system is in place" may need to be reexamined. In many cases, the planned shut in of a well may provide for a safer means of well control than diverting the well, especially after cement is in place.

Current regulations require waiting on cement for 12 hours before drilling out all casing strings other than the conductor casing, and that cement have a minimum of 500-psi compressive strength in the bottom 500 feet. This waiting time does not specifically apply to the nippling down of the diverters. In 3 of the above 4 cases, diverters had been partly nippled down in substantially less time than 12 hours after the plug was bumped. It was fortunate for the rig crews that the diverters were able to be reconnected before the flow became prohibitive. The proper amount of time to leave the diverter system in place is dependent upon the time required for the cement to develop adequate strength to prevent gas flow. (Flows began in an average time of 4.5 hours and a maximum time of 10.5 hours after the plug was bumped in 13 of the incidents for which information was available.)

A better determination of the time required for waiting on cement would be based on lab tests of the cement properties rather than time alone. The Minerals Management Service plans to investigate the issue of waiting on cement. The goal is to better understand well bore cement properties, particularly compressive strength, and how they can be used to establish an acceptable criterion for waiting on cement. This could lead to a regulatory change that better recognizes cement performance criteria and the importance of when well control equipment is nippled down in lieu of the rigid waiting on cement time prior to drilling out of casing in our current regulations.

Flow Incident Example 3

Safety Alert No. 210						        Contact: Lee Fowler

January 8, 2003                        				            979-266-1004

# Blowout After Cementing Surface Casing

While drilling a well, an operator noted a slight gas flow coming from the annulus between the surface and conductor casing shortly after cementing the surface casing. The diverter was closed and pressure started increasing on the annulus. Attempts to bleed off the pressure were unsuccessful. Two days later, gas belched from the drive pipe of an adjacent well. Soon thereafter, gas bubbles were noticed coming from other wells at the water line. The platform was safely evacuated, and the flow eventually ceased.

The subsequent MMS investigation of the incident noted the following conditions encountered by the operator while performing the surface casing cement procedure:  a delay in initiating the cement job because of a top drive electrical problem, a temporary loss of returns while pumping cement, and premature cement returns while pumping the cement.

It has been concluded in part by the MMS investigation of this event that the well control problem was probably caused by the *regression of the cement density to a seawater gradient* and/or the formation of a channel because of the delay in pumping cement. MMS stresses the importance of and recommends pumping cement as soon as possible after landing casing and circulating at least one casing volume.

Other aspects of this incident are currently being studied by the MMS.

* Note:

Scientific evidence in the form of downhole annular pressure measurements

inside the cement column shows that “cement density” or the equivalent

hydrostatic pressure may decrease to values below the seawater gradient.

4.2  Mechanical Barriers

4.2.1 Definition

A mechanical barrier is defined here as “A verifiable seal between two casing strings or a casing string and the annulus that isolates all hydrocarbon-bearing zones below the BOP or diverter achieved by mechanical means”. This seal must be installed, energized, tested and/or verified prior to nippling down (ND) the BOP or diverter. The term verified or verifiable refers to those unique circumstances where the seal can not be tested (such as an ECP packer in open hole). In this case you would have to rely on surface and down hole indications that it is set and in place.

4.2.2  Characteristics and Capabilities

Many annular flows occur while BOPs or diverters are nippled down just after cementing shallow casing strings. This results from: 1) loss of hydrostatic pressure as the cement column begins to develop gel strength and support its own weight, 2) reduced fluid density in the annulus during cement washout operations, 3) gas migration within the unset cement column, and/or 4) a combination of these.

Mechanical barriers can significantly increase the odds of preventing flow after cementing and can also save significant rig time by allowing the operator to move straight into nippling down the BOPs without waiting on cement (WOC). This may serve as an incentive for using a mechanical barrier. Mechanical barriers are still not intended to replace the primary control, a properly designed and well executed cement job coupled with adequate time for the cement to cure. Rather, they serve as a secondary barrier to keep the well from flowing after the cement is in place. A properly designed cement job is extremely important, but there are many factors that can prevent even a well-designed cement job from being properly executed in the field. Examples of these are lost returns, shut downs caused by equipment malfunction during the cementing operations thus preventing uniform and timely placement of cement, getting cement from the wrong tank, etc. Even if there is a mechanical barrier in place, there is still no guarantee that flow around the outside of the shallow set casing shoe to the surface could not occur, but the odds of a flow shutoff between strings are much better.

The industry has successfully used for many years liner top packer to eliminate / reduce flow after cementing on deeper strings as well as squeeze work on liner tops. The liner top packer is a proven product. Devices similar to this or that functionally serve the same purpose (mechanical barriers) can be used on shallower strings. Mechanical barriers will keep the well from flowing, which will allow the cement column to hydrate and cure in a static condition enhancing the probability of zonal isolation. In addition, a mechanical barrier may prevent flow up between the casing string and the BOPs making the job safer.

If the well was static before the cement job, then holding the fluids in place provides the best opportunity for preventing gas from migrating up, and ultimately breaking down the shoe and broaching to the surface. Experience has shown that not having a mechanical barrier in place can result in flow after cementing.

If washing out between strings is required for a given well plan and the probability that hydrocarbons will flow (i.e., the well contains hydrocarbon bearing zones behind the pipe) then a mechanical barrier may be recommended since the effectiveness of the BOPs is negated by the tubulars run inside the BOP or diverter to wash the cement. If a mechanical barrier cannot be used in this situation, then the operator would not be able to wash out.

4.2.3  Subsurface Mechanical Barriers

Some mechanical barriers can be run downhole to isolate an annulus. The positioning of these devices depends on both purpose and design. To avoid exposing potential trapped pressure, the subsurface mechanical barrier should be placed deep enough below the mudline so that it remains in place when the well is plugged and abandoned. The following are examples of items / tools that can be used as subsurface mechanical barriers:

- Inflatable External Casing Packers (ECPs) are packers that have inflatable elements mounted on mandrels that are equivalent to the liner or casing string. The advantages of ECPs include the ability to run through reduced IDs and seal in larger ID sections of casing or irregular open hole. If the ECP is set directly above the problem zone in open hole then all casings above the zone are protected. Elements can be designed for specific applications but the minimum OD is generally 1” greater than casing OD. The wellhead and BOP stack must be selected with a minimum ID sufficient to accommodate the ECP. The differential pressure capability of a liquid filled ECP is a function of element design and the ID of the casing or open hole that it is set in. Inflation with fluid that is inside the casing at the time of the primary cement job is a simple operation that requires only that a differential pressure be applied to the ECP valve system. If the element is inflated with cement then the differential pressure capability is limited only by the casing or open-hole strength. Cement inflation is recommended for many applications. In conventional casing cementing operations spacer fluids are used to locate cement inside the ECP at the time of plug bump and pressure is applied to inflate the ECP with cement. In all applications planning is critical, and analysis of temperature and pressure changes during the life of the well must be included in the ECP design.

- Another mechanical device is the external mechanical casing (EMC) packer and collar combination. Basically a sub is run in the previous casing string with a load shoulder built into it which is fluted for fluid by-pass. This load shoulder will reduce the ID of the casing string which will alter normal bit selection when drilling out of this outer casing string. The EMC packer is run externally on the inner casing string. The inner string must be planned to set off bottom (rat hole below the casing string). Mudline suspension cannot be used with this system because the pipe must be landed off bottom, as well. At the completion of the cement job, weight is slacked off energizing the EMC pack-off on the inner string and creating a seal between the strings. The pack-off should then be tested. This device is not applicable for all casing configurations. It should be sized for the diameters of the tubulars involved and the differential pressure across the annular area. Past performance shows that this tool is effective in preventing annular flow and Sustained Casing Pressure (SCP).

- Mudline Casing Suspension Systems (MCSS) can also be used if cement removal in the annulus to a point below the mudline is desired. This equipment allows for removal of cement without disabling the BOPs. Removal of the cement can be accomplished by opening the wash ports provided at the disconnect point and circulating the cement from the annulus. This is not a strictly mechanical barrier, however. MCSS provide for a means to support the casing string weight at or near the mudline and provide for a means to disconnect the casing strings above that point for the purpose of temporary abandonment. The suspension/disconnect point is often positioned sufficiently below the mudline to accommodate any requirement of the appropriate regulatory agency to remove the casing below the mudline for permanent abandonment. To ensure the ability to disconnect the casing for temporary abandonment, cement should be washed out of the casing annulus above the disconnect point. If water is used, the hydrostatic pressure above the MCSS could be reduced enough to permit the well to flow, and a shut-in would be required. Consideration should be given to the use of heavier fluids to wash out. Shut-in time would also be required prior to nippling down to prevent flow if there is no mechanical barrier used in conjunction with the MCSS.

4.2.4  Surface Mechanical Barriers

Other items can be installed at or near the surface that can serve as mechanical barriers. These devices also seal the annulus, but in the wellhead area rather than downhole. Examples of these are:

- A Multibowl wellhead system provides several casing suspension and sealing positions in a single wellhead housing. This allows the landing and sealing of more than one casing string without the need to remove the BOP after cementing to add an additional well head. A “Multibowl System” can also serve as a mechanical device to seal the annulus and allow the BOPs to be nippled down without waiting on cement for long time periods. The Multibowl head can be installed on top of the drive pipe and used to seal the annulus between the surface casing or conductor casing. Since the inner string is landed off bottom with only the drive pipe to support it the possibility exist for the entire assembly to fall due to the weight of the surface or conductor. In this case; drive pipe should be driven to refusal or into a competent formation. Alternatively, the Multibowl can also be installed on the conductor and rest on the drive pipe for additional support. The Multibowl system may have a retractable load shoulder to land the casing on via a fluted hanger, but there are other types that do not require retractable load shoulders.  The pipe is cemented, and then the hanger and packoff are landed through the BOPs sealing the annulus. The BOPs could be safely nippled down after this seal is tested. All strings must be landed off bottom, which precludes the use of a MCSS.

- Conventional wellhead equipment requires that the BOPs must be disconnected and lifted to install the slip and seal assembly around the casing just cemented. Unfortunately, in most cases, this requires several hours during which the cement column begins to develop gel structure, reducing its ability to transmit hydrostatic pressure to the formation and allowing the possibility of an influx of fluid or gas from the formation. Review of past incidents of flow after cementing revealed that the earliest flow after cement was in place was 2 ½ hours. If the slip and seal installation along with the BOP nipple down could be performed during the time the cement column is still completely fluid, conventional wellhead equipment could be installed with reduced risk. . There is specialized equipment that can facilitate completing this operation in less than two hours. An example is a “Fastlock Connector”. This device does not require studs and nuts (the removal and re-installation of which requires significant time). If the well begins to flow, the BOPs can be lowered and the connection to the wellhead restored quickly. A conventional studs and bolts nipple down would be difficult to execute in the time required. In the event the well began flowing prior to setting the slip and seal assembly, the stabbing and securing of studs would also be a high risk operation.

4.2.5  Alternate Methods

Several other mechanical methods and materials can be employed to reduce WOC time by preventing annular flows. The following are examples of these:

- Scab liners can be run and cemented inside previous casing (drive pipe or surface casing) to save the cost of a wellhead and to save nippling up / nippling down (NU / ND) time associated with the BOP and diverter. In many cases this liner is simply set on bottom inside the drive pipe after cementing in place to seal the annulus. To be effective, cement must come up between the liner and the host string of casing and should be verified with a test. If cement or a seal is not verified and a kick is taken on the following hole section, then the host shoe would see the pressure and could ultimately break down and  flow to surface on the outside of the drive pipe ( host string).  One scab liner system incorporates a shoulder and seal area welded into the drive pipe that accepts the seals and hanger for the scab liner. The scab liner would be set off bottom. This annulus must be tested 150 psi above the frac gradient at the drive pipe shoe to verify the seal is effective before drilling ahead. This could also be an application for an ECP run on the scab liner and set inside the drive pipe.

- Chemical sealants have been effective in stopping some minor annular flows. One example is a product that uses a pressure-activated polymer designed to seal leaks in wells and severe environment hydraulic systems. The polymer crosslinks only at the point of differential pressure at the leak site. Otherwise, it remains a pumpable liquid. Product like this have been successfully used to seal micro-annulus gas leaks in cement jobs between casing strings by injecting it from the surface.

4.2.6  WOC Decision Tree for Surface Stacks

4.2.6.1  Waiting Time Guidelines Prior to Nippling Down

This WOC Decision Tree and guidelines for surface stacks is an excellent tool to use in making a decision on when and under what conditions it would be safe to proceed ahead with ND the BOP or Diverter system. This document is to be referenced whenever there are hydrocarbon bearing zones or the possibility of hydrocarbon of hydrocarbon bearing zones in the drilled section or if it is unknown. Deviations from this guideline are permissible only with the approval of the proper regulatory body such as the MMS.

There may be other downhole conditions that could modify the guideline. It is extremely important that your plan be modified toward a conservative approach to avoid loss of well control. Examples of this are as follows:

- Complete loss of returns while pumping cement.

- Delays in getting cement in place, such as pump problems or getting cement out of tanks that could lead to channeling and poor cement jobs.

- Premature returns of cement to surface.

- High gas units in the drilling mud prior to cementing

In these cases confidence in the cement job would decrease and waiting on cement for 10 hours before nippling down is deemed prudent.

This guideline is based on the best information and technology to date and will be updated as new technology and information become available. In can not be overemphasized the importance of planning for this contingency well in advance of the drilling of these hole sections. These discussions should include all parties involved in doing the work and specifically should include the operator, drilling contractor, cement contractor and the contractor providing the mechanical barrier if applicable.

4.2.6.2  Elaboration on Circumstances

In instances where a hydrocarbon bearing sand is expected to exist in the zones through surface pipe, the operator is required to submit a plan for mitigating the possibility of the zone flowing before, during, or after cementing. If the existence of, or potential for, hydrocarbon zones is unknown or found unexpectedly, a plan on how it will be handled is required and must be approved in progress. Plans can vary depending on a given situation, but should cover the drilling, cementing, and after cementing time frames.

The casing strings that are of primary concern are the drive pipe (example 36” or 30”) and conductor annulus (example 20”, 16” or if conductor has been eliminated it could be 13 3/8” or 10 ¾” for example). The second annulus of primary concern is the conductor by surface pipe annulus (20” x 13 3/8” or 16” x 10 ¾” are examples of common sizes used in the GOM). Usually, you are going from a large annular / diverter system to the 13 5/8” high pressure BOPs which requires the removal of one to install the other BOP or to install a wellhead. The greatest concern in both of these cases is the untimely removal or disabling of the BOPs just after the cement job which is the most dangerous time due to the potential loss of hydrostatic which results as part of the cement curing process. Some people may consider this a safe time since a cement heavier than the drill mud has been pumped into place, but this is just untrue. During the curing process, the equivalent hydrostatic pressure previously created by the cement weight can fall far enough to create an under balance and allow hydrocarbons to enter the well bore which could lead to a disaster if the BOPs are partially disabled by tubulars run along side the casing or BOPs not in place at the right time to prevent fluid movement.

There are other equipment issues. One is the use of mudline hanger systems (MHS) which aid in the abandonment and or a future tieback when the well is temporarily plugged and abandoned. Removing the cement can be done by exposing ports on the inner string at the MHS hanger and circulating out the cement above, spotting sugar water to inhibit the cement from setting up, and then closing the ports and displacing (bullheading) the sugar watered mud below the MHS a safe distance. If you are not using a MHS, then to clear the desired annulus is accomplished in many cases with tubing strings run in the hole just after the cement job through the annular / diverter system to the desired depth below the mudline and then displacing the annular fluid and cement to the surface as soon as the cement is in place. The problem here is that the BOPs are disabled in the event the well flows after cementing and you have a potential disaster on your hands if you needed to shut them in. If there is an exposed hydrocarbon bearing sand, then this practice must be avoided unless there is a mechanical seal between the strings (such as an external casing packer) that could prevent a flow. In any case, when there is a hydrocarbon bearing sand, nippling down without a mechanical seal between strings must be delayed until the cement is past the critical phases in curing where the equivalent hydrostatic could drop allowing the well to flow. Even when it is past this stage, Nipple down through nipple up of the new well head and BOP must proceed as efficiently as possible and with a plan to minimize the exposure time and plan for the unexpected.

4.2.6.3  WOC Decision Tree Procedures

The WOC Decision Tree defines minimum waiting times prior to beginning Nipple down operations of the BOP or Diverter system. In ALL cases, the operator and contractor must have a plan to minimize the time from the start of ND the BOP to securing the exposed annulus with slip and seals. No other activities should take precedence over securing this annulus. As previously stated Waiting time can be reduced or avoided if the operator has a “mechanical barrier” in place and tested prior to ND the BOP. A mechanical barrier is defined here as “A verifiable seal between 2 casing strings or a casing string and the annulus that isolates all hydrocarbon bearing zones”. This seal must be installed, energized, tested and / or verified prior to BOP or diverter removal or BOP impairment.

Step 1:

Determine whether hydrocarbon zones exist in the open hole section of the wellbore.

If it is KNOWN by using MWD/LWD/wireline logging tools that no hydrocarbon zones

exist in the open hole, or that the zones identified have low permeability with little to no flow potential, it is not necessary to wait-on-cement after bumping the plug. In this case, go to Step 4.

If the presence of hydrocarbon zones cannot be determined within the open hole section of the wellbore, or have been found but with unknown flow potential, go to Step 2.

Step 2:

Once cement pumping is complete and the plug has been bumped, waiting-on-cement time required prior to nippling down BOPs is determined as follows (See Note 1. If this is the case, go to Step 4):

1st Level Decision :

Is there a mechanical barrier in place to contain a potential flow between the string or in the form of a special wellhead? If the answer is yes and this seal has been verified or tested, go to Step 4.

2nd Level or Primary Cementing Decision (Use if the 50-psi compressive strength data is known):

This method for determining waiting-on-cement time is when the in-place cement reaches 50-psi compressive strength (See Notes 2 and 3). This WOC time value exceeds the time for obtaining a 500-lb/100-ft2 gel strength, therefore erring on the safe side. Go to Step 3.

3rd Level or secondary Decision (Use if compressive strength data is unknown):

If unable to obtain compressive strength data for the proposed cement slurry or if

unable to confirm NO hydrocarbon zones in the open hole section of the wellbore, wait-on cement time is to be 10 hours plus 2 hours for each 1000’ below 5000’. (See Note 4). Go to Step 3.

In all cases above the WOC time assumes that the cement job went as planned. That is there was no lost circulation, the cement was mixed as designed within a reasonable time, no pump failures causing delays in placement, etc. If not, the operator will wait THE MAXIMUM time before beginning ND operations.

C

Step 3:

Once the wait-on-cement time in hours is obtained for the specific cement slurry, you MUST wait-on-cement for that amount of time prior to removing or disabling the BOP equipment, or have a mechanical barrier seal in place and tested or verified on that annulus.

Step 4:

After making sure the annulus is full, proceed according to the planned procedures of the

Operator (may include nippling down the stack).

Notes:

1) Nippling down when waiting-on-cement becomes a non-issue when the well head or casing annulus can be and has been sealed off and tested. Obviously the rig time involved is expensive and sealing the annulus after cementing would allow the operator to proceed immediately to nippling down the BOPs. Examples of this are:

-  Running the string as a liner with a liner packer and setting it after the cement job

- External Casing Packer (ECP) run between pipe strings.

- EMC packer collar with landing seat on previous casing string

- Multi- bowl system NU on drive pipe or conductor allows casing to be landed and

packed off without nippling down larger diverter / BOPs.

Since 3 of the above require the casing to be set off bottom, mudline suspension systems (MLS), used to facilitate temporary P&A, needs to be thought out carefully because it could preclude the use of any of the 3. The only one that would work on bottom would be the ECP. The pack-off that is run between strings, needs to be run deep enough that abandonment does not remove it; otherwise we might suddenly be exposed to a flow.

Advanced planning is a must between the operator, drilling contractor, and cementing company, and mechanical barrier company if that route is selected.

2) The primary decision information should have been requested from the cementing company long before the cement job commences, preferably at the pre-spud meeting or before. It may be possible for the operator to modify the slurry mixture and have it re-tested to minimize the WOC time.

3) The cement companies report that once the cement reaches 50-psi compressive strength, gas generally can no longer migrate through the cement slurry. This 50-psi compressive strength value is therefore conservative and should take into account variables that can not be reproduced in the laboratory. However, the reported compressive strength times may be in significant error if the temperature used for testing differs significantly from the zone of interest actual temperature. Higher testing temperatures reduce the reported compressive strength set times. This situation may lead to nippling down early and should be questioned.

4) This value was chosen based on thousands of cementing tests conducted in the laboratory by one of the primary cement suppliers. Test results indicate that the vast majority of cement slurries reach 50-psi compressive strength well within the 10 hour time frame. As can be seen, it is much better to obtain the proper cementing information and KNOW when the cement slurry reaches the appropriate WOC value rather than resorting to a rule of thumb measurement.

C

4.2.6.4  Nitrogen Aerated Cement (Foam Cement)

Special considerations will be taken when utilizing foam cement on Jackups.

1) Operators and contractors are required to do a complete risk assessment prior to utilizing foam cement on jackups, and will make sure that the results of this assessment are communicated in the cementing plan.

2) Operators and contractors are prohibited from the running of tubing in the annulus between the casing and the diverter, or BOP, prior to completion of cementing operations and determining that the well has no potential for flow. Any deviation from this will be on a case by case basis by MMS approval

3) The operator will verify that hydrostatic pressure calculations are performed to ensure

sufficient hydrostatic head throughout the well prior to washing out to the mud line

suspension hanger (MLSH). As previously discussed, if there is potential for flow, then washout can not be done without a mechanical seal in place, or the proper time has elapsed to insure the cement has gained sufficient strength to preclude flow after cementing.

4.2.6.5 WOC Decision Tree

WOC Decision Tree for when to ND Surface Stacks

Are there hydrocarbon bearing zones in the open hole ?

Is potential flow contained by a “Mechanical barrier” between strings or special wellhead ?

Is 50 compressive strength data available for this cement ?

* WOC 10 hours minimum, then proceed according to Operator’s plan

Is the 50 psi compressive strength WOC time greater than 10 hrs.?

Yes or don’t know

No

No or don’t know

Proceed according to Operator’s plan (see step 4)

No

Proceed according to Operator’s plan (see step 4)

Yes

WOC specified time,  then proceed according to Operator’s plan

Start *

NOTE: WOC should be 10 hours prior to ND BOP if there is lost circulation during pumping of cement, gas units don’t drop to background, or if there are other significant  problems with cement job.

* If the casing string is set deeper than 5000’ add 2 hours WOC for each additional 1000’

Yes

No

Yes

5.0  Cementing Practices and Factors Affecting Cementing Success

5.1  Introduction

This section summarizes many of the key drilling issues that affect the quality of a primary cementing operation. This section is not exhaustive, nor does it provide the reader with a comprehensive set of detailed recommendations for well construction. The intent is to highlight the salient aspects that should be considered and summarize the interrelationship between drilling operations and cementing success. All topics discussed are covered in detail in various API, ISO, and industry publications.

5.2  Hole Geometry

Hole geometry affects many aspects of the cementing operation. Among these are centralizer/centralization requirements, casing running conditions and ability to move pipe, mud removal, and equivalent circulating density (ECD) while circulating and cementing.

The relationship between hole size and casing size should, from a cementing perspective, always strive to achieve the best balance between displacement efficiency and equivalent circulating density. Past studies have indicated that 0.5 inches to 0.75 inches are generally recommended minimum clearances (hole size 1 to 1.5 inches > than pipe size). However, modern fluid systems and fluid mechanics modeling allow more precise matching between hole geometry and fluid properties. Maximum recommended clearances between casing and the wellbore should be determined by desired minimum annular velocity and fluid friction pressure profiles versus available hydraulic horsepower and/or maximum pump rates as determined by surface equipment and open hole fracture gradients. Minimum annular clearance is determined by drilling and completion objectives, how much annular coverage is desired, formation limitations (fracture versus pore pressure), and logging considerations.  Cement sheaths less than 0.5 inches thick are difficult to evaluate with CBL.

Drilling and fluid management practices should strive to minimize hole washout via proper bit hydraulics and shale inhibition. Weight on bit and rate of penetration should be controlled to minimize unintentional doglegs and/or trajectory changes. Under-reaming practices and bi-centered bit use should concentrate on minimizing double-helix hole geometries that interfere with mud removal and pipe movement during cementing operations. The directional survey, complete with azimuth, should be included in centralization, torque & drag, and displacement modeling, especially when wellbore spiraling cannot be avoided.

5.3  Drilling Fluid Type

Drilling fluid (mud) selection and maintenance play a key role in cementing success. Drilling fluid performance affects hole condition (washouts), mud filter cake thickness and gel strength, mud mobility, fluid and formation compatibility, and bonding of cement to formation.

Drilling fluid selection is generally dictated by anticipated formation types and drilling conditions, penetration rate and differential sticking and environmental, disposal and logistical considerations. While drilling fluid selection does impacts subsequent cementing operations, that impact is not often considered in the selection process. Unfortunately, the ideal drilling fluid is not necessarily that best suited for cementing operations.

Drilling fluid performance is controlled by many factors.  Low-gravity or active solids should be adequately controlled, and adequate shale inhibition should be provided for aqueous mud systems. HTHP filter cakes should exhibit thin, low permeability characteristics.  Static gel strength and low shear rheology at HTHP conditions should be low and nonprogressive, but not at the sacrifice of drill cuttings or barite support.

Achieving good cementing success through effective mud displacement requires proper preplanning.  Computer modeling of cement placement or mud displacement requires careful evaluation of fluid properties and placement processes.

Rheological properties of the drilling fluid should be obtained at anticipated wellbore temperatures and pressures for proper displacement modeling. The rheological properties of some drilling fluids, and synthetic–based fluids in particular, can vary widely with changes in pressure and/or rheology. Drilling fluid samples representative of the drilling fluid at the time of cementing should be tested with cementing fluids for rheological and chemical compatibility. Laboratory-prepared samples of drilling fluids are generally not sufficiently representative of actual well fluid for this evaluation. Samples collected at the rig as near to casing set point as logistics permit should be used for these evaluations.  In the case of nonaqueous drilling fluids, the ability of the cementing spacers and/or preflushes to fully water-wet pipe and wellbore should be evaluated as detailed in API RP 10B and ISO 10426-2.

5.4  Casing Hardware

Casing hardware and auxiliary downhole equipment may be used to enhance the cementing operation. This includes centralizers, float equipment, stage collars, external casing packers, liner hangers and liner top packers and expandable casing.

Centralization of the casing is of paramount importance to successful zonal isolation. Computer simulators are highly recommended for determining the best centralization design for a given application. Caliper logs are highly recommended since they can be used to improve the accuracy of centralizer and cementing simulations, and evaluate hole condition and drilling performance. Two-arm calipers are acceptable, but four-arm caliper logs are preferred.

Casing centralizers exist in many models and designs and are generally categorized as either bow-spring or rigid models. Auxiliary functionality such as flow diversion and mechanical friction-reducing methods are also available on both types. Minimum performance specifications for bow-spring centralizers are provided in API Specification 10D and ISO 10427-1. However, it is recommended to use the actual standoff performance specifications for each specific model as provided by the manufacturer. Custom-built centralizers are available for either slimhole or extremely large annular clearances.

Centralizers can be installed such that they are allowed to slide between casing collars or clamped in place via either limit clamps or set screws on the centralizer itself. Such holding devices must be of adequate construction to withstand the forces of running casing and pipe movement during cementing. Centralizers installed on flush joint casing must be equipped with stationary holding devices.

Float equipment is used to prevent the cement from flowing back into the casing when pumping is stopped and/or pressure released. Test procedures for various classes of float equipment and associated mechanical components are provided in API RP. Float equipment choice should always be matched to anticipated bottom hole temperatures and the back-pressure expected at the end of the cement job. Specialized float equipment including auto-fill, side-ported, and custom-profiled is also available to address the functionalities of surge-reduction and collapse protection, improved hole cleaning, and ease of running casing, respectively. Float collars are also available that lock and hold the wiper plugs to prevent rotation during drillout. A float collar and a float shoe can be used together to provide redundant back-pressure control as well as a trap for contaminated cement slurry. Some lost-circulation materials (LCM) in the drilling fluid and/or cementing fluids may preclude or hamper the use of certain float equipment valves designs.

Casing and liner wiper plugs, whether conventional surface release or subsea release (SSR) provide the function of mechanically separating cementing fluids from the drilling fluid, wiping the ID of the tubulars, and providing a positive indication of the end of displacement. Wiper plugs should be matched to the anticipated bottom hole temperatures, pressures, depths and mud type (aqueous or nonaqueous). Not all wiper plugs and associated operating systems (releasing darts) are compatible with all types and manufactures of float equipment and/or stage tools, liner hanger tools, and liner top packers. Tapered casing strings may require customized wiper plug systems. Wiper plugs and associated operating systems should be matched to the casing size and weight and all other casing hardware components to ensure compatibility.

Cementing plug containers and head assemblies provide the means by which conventional casing wiper plugs can be launched from the surface without substantial interruption in pumping operations. All types and manufactures of cementing heads have their own specific capabilities in terms of pressure ratings, mechanical operation, load-carrying capacity, and auxiliary capabilities such as allowing rotation or remote operation. Not all casing wiper plugs are compatible with all models of cementing heads and float equipment. Cementing heads and plug sets should always be checked for compatibility. Plug containers equipped with remotely-operated plug releasing mechanisms reduce risk to personnel and may minimize shut-down time.

Cementing heads should always be in good working order and be pressure-tested on a regular basis. Various components that should be examined and tested on a regular basis are the cap, coupler, seals, O-rings, extensions, threads, lift swivels, hoist rings, manifolds, along with any associated swages. Magnetic particle inspection should be performed on a periodic basis, or if a component is suspect. Type-Certification is available from the service provider in many instances. A Type-Certified Plug Container is one that has fully traceable components with material mechanical properties verified by laboratory tests. In addition, a Certification of Design Strength Approval for the design is issued by Lloyd’s Register.

5.5  Close Tolerance Considerations

Close tolerances are encountered in pipe-to-pipe annuli, liner tops, expandable casings, and between the inner diameter of outer tubulars and some tools such as liner top packers, stage tools, external casing packers (ECP’s), and expandable tubulars. Close mechanical tolerances should be examined for mechanical (drift) compatibility with all components involved in the cementing operations including wiper plugs and associated operating components. Minimum and average cross-sectional area and length of the restricted flow path should be examined for compatibility with drilling and cementing fluid properties and/or material components such as LCM. Circulating pressure calculations should include any flow restrictions associated with close tolerances, particularly those of significant length such as liner laps and tieback sleeves.

5.6  Engineering Design

Cementing objectives determine the cement designs to be used, extent of cement coverage with each, and performance requirements of each.  Performance requirements include gas control, fluid loss, free fluid, thickening time, and compressive strength.  Tensile strength and elasticity may also be considerations.

Zonal coverage determination. It is important to evaluate which zone(s) have potential for flow in order to plan the cement job to achieve suitable zonal isolation. Ideally, such zones should be covered with a cement slurry designed to prevent flow after cementing and the cement placement mechanics should be designed to maximize mud removal. Zones left uncemented may not flow in the short term if pore pressure is balanced by mud hydrostatic head. However, phenomena such as barite sag and mud dehydration may lead to longer-term sustained casing pressure.

Cement top selection is influenced by the location of the problem zones, regulatory requirements and pore pressure/fracture gradient considerations. Higher density tail slurries may be more easily designed to be “gas tight” than lighter weight lead systems.

If the potential problem zone is to be covered by tail cement, the tail slurry should have a shorter zero gel period than the lead slurry so gel strength development in the lead slurry does not reduce the hydrostatic head exerted on the zone before the tail slurry goes through transition.

Pore pressure/frac gradient. Accurate knowledge of pore pressure and fracture gradient profiles is necessary for a successful cement job. Pore pressure is one piece of information needed to assess flow potential. The pore pressure and fracture gradient profiles are two of many input values used in computer simulation programs used to evaluate static and dynamic well security.

Temperature (Thermal modeling and/or measurement or offset) Temperature is the single greatest influence on cement slurry performance. Accurate estimates of cementing temperatures (both static and circulating) are essential to the success of the cement job.  For many wells, API temperature schedules provide adequate estimates of circulating temperatures.  However, these schedules are based on data collected in wells in shallow water with little deviation (API Report 10TR3 - Technical Report on Temperatures for API Cement Operating Thickening Time Tests – 1993 Report from the API Task Group on Cementing Temperature Schedules).  The API schedules should not be used for wells that vary greatly from these conditions (e.g. deepwater).

For such “non-standard” wells, circulating temperature information can be obtained using temperature recording devices that are made up in the drill string or dropped into the drill string and run on clean-up trips. These devices measure the maximum temperature encountered as well as changes in temperature as the well is circulated. The measurements are taken with a geometry (drillpipe) different than what will be present in the well during the actual cement job (casing) so the measured values may deviate from values during cementing. Also, because the devices are run after the hole section has been drilled, little time is available for extensive slurry modifications. Finally, temperature measurements are more commonly considered in production hole sections than in shallower strings, although the affects of temperature on cement performance are just as important in the shallower strings, especially if there is potential for flow.

Computer-based thermal modeling programs may be used to develop cementing temperatures. Such programs are not simple and require a relatively large amount of input information including static temperature, formation and well fluid thermal characteristics and estimated job volumes and pump rates.

Temperature information may be obtained from measurement-while-drilling (MWD) tools as the hole is drilled. Temperatures from MWD devices tend to be higher than those derived from models or sensors used on cleanup trips. Several things account for this including an elevated mud suction temperature while drilling compared to initial slurry temperature while cementing. Some bottomhole assemblies and formation types have been demonstrated to give very elevated MWD readings.

In summary, there are many sources of temperature information. All temperature information should be considered.  Rather than a single, “correct” answer, the engineer will likely be presented with a range of likely temperature values. The slurry design must perform acceptably over the anticipated range.

Mud removal design. Proper slurry design is only part of a successful cement job. The other part involves efficiently removing the drilling fluid from the well and replacing it with cement slurry. Computer based placement simulators allow the engineer to tailor the cement job to a particular well’s conditions rather than relying on “rules of thumb”.  Factors to include in planning for efficient mud removal include:

Pump rate. Higher pump rates introduce more energy into the system allowing more efficient removal of gelled drilling fluid. However, tight annular clearances in some wellbore configurations limit how fast the job can be pumped without causing lost returns.  Depending on fluid rheology and degree of centralization, pumping too fast may actually lead to uneven flow between the narrow and wide sides of the annulus. Computer simulators allow us to consider many factors when determining optimum pump rates.

Fluid types. The types of fluids used during drilling and cementing operations affect the cement job. A drilling fluid conducive to wellbore stability benefits the cement job by producing a uniform, near-gauge hole with minimal hole washouts.

In addition, all fluids used in the cementing operation must be compatible. Compatible fluids are fluids that are “capable of forming a mixture that does not undergo undesirable chemical and/or physical reactions”. Mixtures which become viscous when mixed are difficult to displace efficiently and may cause lost circulation due to excessive friction pressure. If non-aqueous drilling fluids are used, a spacer containing surfactants that water wet downhole surfaces will be necessary (compatibility and wettability test procedures are found in API RP10B)

Rheology and density. As a general statement, mud removal is more efficient when the displacing fluid is “thicker” and heavier than the fluid being displaced. Various guidelines have been used to decide how much heavier and thicker each step should be in this density and rheology hierarchy. Narrow pore pressure/fracture gradient windows in some wells may limit application of density and rheology hierarchies.

Mud compressibility (affect on density and rheology). Unlike water-based muds, nonaqueous drilling fluids are compressible.  Depending on the pressure, the fluid density may be increased by as much as 0.5 lbm/gal due to compressibility. Compressibility may dramatically increase fluid rheology as well.  Commonly available viscometers used at the rig take measurements at atmospheric pressure and thus will not show the effects of compressibility. For more accurate cement computer displacement simulations, mud samples should be sent to a laboratory for testing under higher, more realistic confining pressure.

Spacer design. The purpose of a spacer is to aid in bulk mud removal by avoiding incompatible mixtures of cement slurry and drilling fluid. When non aqueous drilling fluids are used, the spacer also serves to strip away the oily mud film and water-wetting the downhole surfaces. Procedures for testing spacer compatibility are found in API RP10B. Some computer simulators can help optimize the volume of spacers to be pumped for mud removal.

Pipe movement. One of the best aids to achieving effective mud removal is pipe movement; either rotation or reciprocation.

Centralization.  If casing is not centralized, it will lay near or against the borehole wall. Mud, washes, spacers and cement slurry flow most easily on the wide, less restricted side of the annulus. It is difficult, if not impossible to displace mud efficiently from the narrow side of the annulus. This results in bypassed mud channels and loss of zonal isolation. Centralization is necessary to improve flow all around the pipe and aid in mud removal.

Computer Simulations (displacement efficiency, fluid hydraulics, well security and control). As is true with many areas of study, computers have greatly improved the design process for well cementing. Using computer simulators, the engineer can tailor the cementing process to account for an individual well’s unique conditions. It is no longer necessary to rely on general “rules of thumb”.  Each cementing service company has its own simulator. Simulators are also available from 3rd party vendors.  These simulators vary in their capabilities and strengths. Typical capabilities include:

- U-tube calculations to predict whether the cement job can be performed within the pore pressure/frac gradient window, considering ECD

- Displacement efficiency

- Foamed cementing calculations

- Circulating and post cementing temperature profiles

- Swab and Surge Pressure

As with any computer program, the quality of a cementing simulator’s output depends on the degree to which the input variables are known. It is not likely that a simulator can give you a single “right” answer.  However, by bracketing variables, the engineer can gain insight that will assist in achieving zonal isolation.

5.7  Slurry Design and Testing

Cement must be placed in the wellbore and provide good contact with the casing and borehole wall, prevent the formation of channels within the cement and prevent the invasion and propagation of fluid through the cement as it sets. The cement should maintain its integrity throughout the operation of the well and after.

Cement properties necessary to meet these objectives include

- rheological properties that aid mud displacement,

- hydrostatic pressure control,

- fluid loss control as appropriate,

- free fluid and sedimentation control,

- control of gel strength development,

- resistance to invasion of gas or fluid,

- rapid set and adequate short term and ultimate strength,

- long-term sealing (bonding/flexibility);

Test methods for determining the performance of cement are described in API RP 10B (ISO 10426-2), ISO 10426-3, ISO 10426-4, and ISO 10426-5. These methods should be modified, as closely as possible, to the conditions to which the cement will be exposed during placement across the potential flowing zones requiring isolation. Temperature/pressure schedules should be devised for conditioning and curing the cement for these tests. The use of thermal wellbore simulators is discussed previously.

Lead and Tail Cement – Lead and tail cements are routinely placed in the annulus during primary casing cementing. Lead cements are cement slurries that are designed with lower densities by extending the cement with low cost components, either water or readily available low-cost materials. In addition to the cost consideration, low density lead cements are used to lower the hydrostatic pressures to avoid or minimize losses of the cement to the formations. Tail cements are cements mixed without extending components and are thus more costly.

The design of the cement which will cover the potential flowing formations must be considered carefully. Lead cements, although not normally “designed” to cover formations which might flow, can be designed to control flows. Doing so may require special formulations. In most cases, potentially productive formations which have the potential for flow after cementing are covered by “tail” slurries. Such slurries are frequently designed with special properties and/or additives to control the flow. Lead slurries which are placed across “non-productive” formations having the potential to flow should be designed with the same criteria as slurries placed across the pay zones.

Density - Density plays a key role in the design of cement slurries. In cases with potential for flow, there are two primary considerations for selecting the density: preventing losses to the formation and preventing flow from permeable formations. This implies the density must fall between that necessary to provide enough hydrostatic pressure to control flow from the permeable formations and that which will fracture the weak formations (see previous discussion of pore pressure/frac gradient under Engineering Design).

Other considerations related to the density of the slurry are the performance related to strength development, and slurry stability (see subsequent discussion). Meeting the requirements of strength and slurry stability may make the use of special slurry formulations necessary to achieve the required density.

The density under placement conditions (temperature and pressure) must be considered in the design. Some slurries are compressed by pressure while others have components which are deformed by pressure. Either of these can lead to higher densities after placement downhole than the density at which the slurry was mixed at surface.

Thickening Time – The thickening time is the time that a slurry is judged to be pumpable. Testing should be under conditions simulating those downhole during placement. Slurries should be designed for the specific set of conditions found in the well and for the designed pumping schedule (rates) to be employed during the cementing operation. Wellbore temperature simulators are used to model anticipated well temperatures while cementing and can be used to develop schedules for conducting tests.

The use of excessive safety factors in thickening time design should be avoided. Excessive safety factors can cause delayed setting, long periods of gelation and increased likelihood of solids segregation. These factors may present a higher potential for flow from the formation before the cement has adequate strength to prevent it.

Thickening times should be designed so that slurries set from the bottom to the top of the well. Gel strength development progression should also be from bottom to top.

Fluid Loss – Control of fluid loss plays a key role in preventing flow. Loss of fluid after cement placement is a contributing factor in the loss of the overbalance pressure controlling flow. The rate of fluid loss is dependent on the overbalance pressure, the permeability of the formation, the condition of the mud cake (including its permeability), and the fluid loss characteristics of the cement. Testing should be done after conditioning the slurry at conditions similar to those in the wellbore. Fluid loss test temperature should be that which the slurry will experience across the potentially flowing zones. It is not possible to make a precise rule about the required level of fluid loss; it depends on many factors, but lower fluid loss is always better where there is potential for flow.

Slurry Stability; Sedimentation, Free Fluid – Stability of the slurry is an important property in preventing annular flow. Free fluid and sedimentation may occur simultaneously or one may occur without the other.

Free fluid can result in a channel or a void in the cement into and through which formation fluid or gas can easily flow. It may also result in a severely underbalanced condition (through the water channel) initiating the flow. Control of free fluid is imperative in situations where there is the potential for flow.

Not only is the presence of a channel and the resulting underbalanced condition critical, but the condition of the remaining slurry is key as well. When water is lost from the slurry (by free fluid separation), the solids concentration is increased. This can result in uncontrolled gelation, changing other properties of the cement (such as ability to transmit hydrostatic pressure).

Additionally, sedimentation (which results in concentration of solid particles in lower sections and reduced concentrations in upper sections of the well) must be controlled. The properties of the slurry will be changed by sedimentation, leading to greater gelation where the solids are concentrated and low strength and high permeability where they are reduced.

Slurries should be conditioned at temperatures encountered during placement and free fluid and sedimentation tests conducted at the temperatures to be encountered while static in the annulus.

Rheology – The rheology of the cement slurry is important for two reasons; the displacement of fluids ahead of it and the friction pressure generated during placement. Design should take these into account.

Rheology has a secondary effect on the slurry stability. Gel strength development may also be affected by components of the cement which are used to control rheology. All of these effects must be considered together. Rheology is tested at atmospheric pressure at temperatures up to 189oF (870C). To avoid burns, care must be taken when such testing is conducted at high temperatures.

Gel Strength – Gel strength affects the ability of slurries to suspend their solids. It also contributes to decay of hydrostatic pressure as the gelled fluid interacts with the casing and the borehole wall. Thus, the gel strength of cement slurries should be designed to be adequate to support the solids and yet, not be excessive. One method to evaluate gel strength development is to measure the “critical gel strength period.”

There are two approaches to the control of gel strength for annular flow control; low gel strength until just before setting and rapid gel strength development to 500 lbf/100 ft2.

The Critical Gel Strength Period is the time between the development of critical static gel strength (CSGS) and 500 lb/100 ft2.

The CSGS is the gel strength of the cement that results in decay of hydrostatic pressure to the point that the pressure is balanced (equals pore pressure) across the potential flowing formation(s).

The CSGS is computed by:

CSGS = (OBP)(300)/(L/Deff), where

OBP = overbalance pressure (psi)

300 = conversion factor (lb/in.)

L = length of the cement column (ft)

Deff = effective diameter (in.) = Dc - DOH

Dc = diameter of the casing (in.)

DOH = diameter of the open hole (in.)

The critical static gel strength should not develop before mixing and pumping stoppages (such as dropping a wiper plug) have been completed.

Gel strength is intimately related to the hydration of the cement, the chemical reaction between the powdered cement and water to produce a solid matrix. These reactions and the gel strength depend on temperature, the chemical and physical nature of the cement being used and materials added to the cement to control its properties (e.g., thickening time, fluid loss, etc.). Gel strength can be controlled by the use of additives. Additives for controlling other properties of the cement should be selected with control of gel strength in mind.

There are no standardized tests for gel strength determination. Service companies have developed methods for determining gel strength at temperature and pressure to simulate wellbore conditions. The gel strength can be measured using a device that allows measurement under pseudostatic conditions [at a rotation of 0.2o/minute (or at shear rate of 10-3 sec-1) or less] on an apparatus designed to make this measurement under simulated downhole conditions. The gel strength may also be determined using special rheometers designed to measure gel strength, pressure drop measurements or ultrasonic correlations. It cannot be determined using a consistometer or a standard rheometer.

Critical Gel Strength Period – The critical gel strength period should be minimized to the extent possible, preferably less than 45 minutes (measured at the temperature of the potentially flowing zone).

Strength – The development of strength is a primary consideration in continued operations following a primary cementing job. Early work1 has shown that 8 psi tensile strength (or about 100 psi compressive strength; compressive strength is about 8-12 times tensile strength for most cements) is adequate to support the casing. Waiting on cement (WOC) time is normally until laboratory testing has shown the cement around the casing shoe to develop 500 psi compressive strength. The allowable activity after cementing should depend on the strength development profile of the cement (see additional discussion of WOC under “Post Cementing Operations” and “WOC Tree” under Mechanical Barriers).

There has been a tendency in the past to maximize compressive strength. For the short term, this may seem appropriate, but for later well operation, high strength may be contrary to good well integrity (see discussion under Mechanical Properties).

Compatibility – Any fluid being displaced should be compatible with the fluid displacing it. Fluid compatibility has been discussed previously under Engineering Design: Mud Removal. Compatibility between cement slurries in not normally a problem, but with certain types of slurries it can be. This is especially true for latex-containing slurries in contact with extended slurries. Compatibility between slurries should be tested to ensure that rheological behavior, thickening time, fluid loss and slurry stability are not compromised. Procedures described in API RP 10B, Section 16 can be adapted to test cement slurries for which there is concern about compatibility between the slurries. Rheological behavior, thickening time, fluid loss and slurry stability should be evaluated.

Mechanical Properties – Besides compressive and tensile strength, Young’s Modulus and Poisson’s ratio play a key role in the integrity of the cement during the operation of the well. Stresses placed on the cement by changing pressures (including pressure testing, hydraulic fracturing, etc.) and temperatures in the well or changes due to reservoir pressure can cause a microannulus to form between and the casing or can cause radial cracking of the cement sheath, or both. These conditions can lead to fluid leakage through the cement sheath, sometimes to surface. Pressure at the surface caused by such leaks is commonly known as sustained casing pressure.

High strength cement is brittle and subject to failure under stress. More flexible cement can better withstand stress without failing. Such more flexible cements often exhibit lower, but adequate, compressive strength. Lower Young’s modulus cements are more flexible than those with high Young’s modulus.

Wellbore simulation software can be used to determine the proper combination of strength and flexibility for a specific well within its operation conditions.

There are no standardized tests for determining Young’s Modulus and Poisson’s ratio for cement under wellbore conditions. Tests have been adapted from other disciplines for testing oil and gas well cement formulations.

Flow Control – Proprietary slurries are available for cementing where there is risk of annular flow. Service company design criteria should be met for use of these slurries.

Expansion/Shrinkage – Cement may shrink under certain conditions encountered in the wellbore. Such shrinkage may lead to loss of seal, leading to leakage through or around the cement. Expansive agents may be employed to counteract the effects of this shrinkage.

Test methods for determination of shrinkage or expansion of well cement are under development (ISO 10426-5, Petroleum and natural gas industries – Cements and materials for well cementing – Part 5: Test methods for determination of shrinkage and expansion of well cement formulations at atmospheric pressure).

Foamed Cement – Foamed cement is sometimes used for control of annular flow due to the ability of its compressible phase to maintain pressure in spite of gel strength development. An additional benefit of foamed cement is the greater flexibility of the set foamed cement. Many of the same criteria for design of unfoamed cement are key in designing foamed cement for oil and gas well operations. Thickening time, slurry stability, fluid loss and strength development should be designed for the conditions of the well. High permeability foamed cements should be avoided. In addition, the effects of pressure on the compression of the gas phase and consequently the density must be considered.

An operational benefit of foamed cement is the ability to easily change slurry density by adjustments in the gas ratio. Care must be exercised when making such adjustments to ensure that other design criteria are not compromised (strength, permeability, etc.)

Foamed cement requires special testing methods. Due to the compressible nature of foamed cement, standard well cement testing methods cannot be used for the foamed cement itself. Tests on the foamed cement are made at atmospheric pressure only, unless special equipment is used. Atmospheric pressure testing standards are being developed for foamed cement (ISO 10426-4, Petroleum and natural gas industries – Cements and materials for well cementing – Part 4: Preparation and testing of foamed cement slurries). In addition to procedures for generating the foamed cement, methods for testing foam stability, compressive strength and permeability are described in that standard. Determination of the thickening time, fluid loss and rheological properties are determined for the base cement slurry.

5.8  Wellbore preparation and conditioning

Every effort should be made to minimize the time between completion of the hole interval and cementing when flow hazards exist. With the cementing process in mind, the fluids used to drill should be designed for ease of removal. Conditions should be maintained to minimize changes in hole conditions which would lead to difficulty achieving a seal during the cementing operation.

## 5.8.1  Well Preparation

Well preparation, particularly circulating and conditioning fluids in the wellbore, is essential for successful cementing.  Many primary cementing failures are the result of fluids that are difficult to displace and/or inadequate wellbore conditioning.  Particular attention should be placed on low fluid loss (thin, tough filter cake) and rheological properties that provide low, flat gel strengths.

Even when good well preparation is planned, contingencies in the cementing operation should be provided in case well conditions prevent the planned well conditioning program from being performed.

Well preparation includes:

- Determining hole size,

- Adjusting drilling fluid rheological properties to aid in its removal during cementing,

- Ensuring the well is dead ,

- Curing losses,

- Conditioning of fluids prior to cementing to ensure that gel strength is broken, that cuttings and gas are removed and that the well is cooled for cementing.

The precementing considerations that are included in this summary are based on sound cementing best practices that are known to enhance the probability of success. Primary cement job failures are predominately due to a breakdown in the "displacement process" which leads to channeling of the cement through the drilling fluid. These guidelines when applied in conjunction with a simulation software program will enhance the displacement process and improve the probability of successful primary cementing. Simulation software can be used to help determine the optimum displacement parameters and safe operating ECD.

- Caliper: A caliper is a necessary part of any cementing design. The whole size must be known to allow proper calculations of friction pressure, both during the cementing operation and when running casing. It is also necessary to calculate the centralizer requirements and from centralizer calculations, to calculate flow regimes and rates required for effective mud removal. The caliper must be of sufficient quality to make the necessary calculations.

- Control Lost Circulation: Before cementing casing or liner, lost-circulation should be eliminated or significantly reduced. This is best done prior to cementing. If this is not possible, or if losses are anticipated during the primary cementing job, there are several options. The first is to maintain the downhole circulating pressures below the pressure at which losses occur by reducing the density of the cement slurry, minimizing the height of the cement column and limiting friction pressure during the cementing operation. Other options are to pump a plugging material as a spacer in front of the cement slurry and add lost circulation materials to the cement slurry itself. In practice, these techniques are usually used in combination.

- Wiper Trip; Conditioning the Drilling fluid: After logging and prior to cementing, a wiper trip should be made to help stabilize the wellbore and remove any remaining cuttings. The mud should be conditioned in preparation for cementing. The condition of the drilling fluid is one of the most important variables in achieving good displacement during a cement job. Regaining and maintaining good mobility of the mud is key. Drilling fluids with low gel strength, low rheology and low fluid loss are more easily displaced. Pockets of gelled fluid, which commonly exist following drilling, make displacement difficult and must be broken up. Conditioning should include adjusting the mud properties to those which will be favorable for mud removal during cementing.

- Flow Rate: Flow rate is dependent on well geometry, fluid densities and rheologies. Turbulent flow conditions are desirable, but are not required. Some modeling indicates using high energy displacement rates can allow fluid to be bypassed if the flow is greater than the maximum allowable. A computer simulation should be used to determine the optimum pump rates to obtain the best flow regime and safe ECD's.

To condition the drilling fluid in preparation for a cement job, perform the following:

- Modify the flow properties of the drilling fluid to optimize drilling fluid mobility and removal of drill cuttings. Yield point (YP) and gel strengths should be maintained as low as possible without causing settling of solids from the fluid. These variables must be evaluated for the specific well conditions. In deviated wellbores a higher-viscosity drilling fluid may be required to help prevent solids from settling on the low side of the wellbore. The presence of large drill cuttings may also necessitate higher viscosity fluids. The higher-viscosity fluid needs to be optimized based on wellbore conditions and inclination. After removal of cuttings, consideration should be given to lowering viscosity of the drilling fluid to aid in its removal.

- Measure gel strengths at 10 seconds, 10 minutes, 30 minutes, and 4 hours to examine the gel strength profile of the drilling fluid. In addition, a longer term gel strength test should be run at the bottom hole temperature and pressure. If gel strength profile is not flat after 4 hours, continue to test to match the maximum planned static time for operations such as tripping, logging, or running casing or liner. This testing would typically be performed during the job planning stage. During conditioning just prior to the job, readings taken at 10 seconds, 10 minutes, and 30 minutes are typically sufficient. Well-conditioned mud should have a 10 second gel strength that is < 15 lbf/100 ft2 and the 10 minute gel should be no more than 1.5 times the 10 second gel or the 10 minute gel strength should be < 15 lbf/100 ft2.

- Decreasing the filtrate loss into a permeable zone enhances the creation of a thin filter cake. A high fluid loss creates a thick or high viscosity, drilling fluid layer immediately adjacent to the formation wall that is difficult to displace prior to or during cementing. The fluid loss required is dependent on the hole section being drilled.  In the surface section the fluid loss (API) is generally not controlled or reduced to <20 cc prior to running casing, while in deeper sections the fluid loss is reduced further.  Fluid loss control should be maintained while conditioning the hole and running casing and cementing. Note that a thick, gelled filter cake deposited while drilling using high fluid loss mud cannot be repaired by later lowering the fluid loss of the mud

5.8.2  Running Casing

The surge effect of running casing can result in fracturing or wellbore damage. When casing is run into the hole, a piston effect is caused by the friction pressure of the gelled mud flowing past the casing as it is run. The mud flow rate (and the friction pressure) is proportional to the casing running speed. Running casing with a closed end (normal float equipment) causes the mud to flow faster up the annulus, whereas when run open-ended or with automatic fill float equipment, the fluid velocity (and friction pressure) in the annulus is reduced since part of the mud fills the casing.

As the well has been static since the wiper trip, the mud will be gelled. This also increases the surge pressure.

a. Surge mitigation tools/procedures

With sensitive or weak formations, the surge pressure must be minimized. Surge pressures can be reduced by reducing running speed and by the use of floats that allow mud to flow into the casing while it is run. Surge pressures can also be reduced by reducing the rheology and gel strength of the drilling fluid prior to running casing.

Surge effects can also be reduced by pausing while running casing to circulate mud, removing the gelled mud from the well and replacing it with well-conditioned mud from the pits.

Casing speeds should be evaluated prior to the job by using surge calculation software to determine maximum running speeds without damaging the formation or losing mud. Surge simulation software can also show the effects of lowering mud viscosity or gel strength and may be used for input to the fluid engineer for use in conditioning the mud during the wiper trip.

- Centralizer program

Centralizing the casing across the intervals to be isolated helps optimize drilling fluid displacement. In poorly centralized casing, cement will follow the path of least resistance; as a result, the cement flows on the wide side of the annulus, leaving drilling fluid in the narrow side. In a deviated wellbore, standoff is even more critical to prevent a solids bed from accumulating on the low side of the annulus. The required standoff should be determined from computer modeling of mud removal and will vary with well conditions.

Centralizers should be run according to an engineering design for optimum cementing results.

5.8.3  Conditioning After Casing is Landed

When the casing is on bottom and before cementing, circulate the drilling fluid to break its gel strength, decrease its viscosity and increase its mobility. Condition the drilling fluid until equilibrium is achieved.

Determine the volume of the circulatable hole and calculate the percentage of the openhole annular section that is being circulated. Good fluid returns at the surface do not reliably indicate the mobility of fluid in the annular space. For best results, use a fluid caliper to determine downhole fluid mobility.

A fluid caliper is a small volume of fluid which is easily identifiable when it returns to the pits after circulation. Knowing the time between injection and  recovery, and the pump rate, the volume of fluid which is flowing in the well can be calculated. A fluid caliper pumped through the well in a full circulation helps perform several functions:

- Measure the openhole circulating volume by subtracting casing capacity and pipe-in-pipe annular volume from the trip volume.

- Measure hole cleaning performance (gelled mud/cake/cuttings removal) of various methods/materials.

- Validate cement volumes predicted by wireline caliper logs.

If less than 95% of the openhole volume is circulating, various methods and materials should be used to clean the hole to achieve at least 95% circulation in the open hole annulus prior to cementing. Hole cleaning methods include higher circulating rates, pipe movement, and the use of high/low viscosity pills to remove any partially dehydrated "gelled" mud, wall cake, and cuttings that can impair mud displacement during cementing. More information can found in the following technical papers:

- Smith, T.R.: “Cementing Displacement Practices: Application in the Field,” SPE/IADC paper 18617 presented at the 1989 SPE/IADC Drilling Conference in New Orleans, La. on February 28 - March 3.

- Griffith, J.E. & Ravi, K.M,:“Monitoring Circulatable Hole with Real-Time Correction: Case Histories”, SPE 29470 presented at the 1995 SPE Production Operations Symposium, Oklahoma City, April 2-4.

At a minimum, condition the hole for cementing by circulating 1.5 annular volumes or one casing volume, whichever is greater.

When the drilling fluid is well conditioned (drilling fluid properties in equals properties at the outlet), continue circulating the drilling fluid until spacer and cementing fluids are pumped. Minimize shutdown time between conditioning and cementing by installing the cement head and pressure testing lines BEFORE conditioning the well. Minimize time to drop plug by proper planning. It is best to land casing close to the floor to allow easy access to pins and valves on the cementing head necessary to drop the plug (and to minimize hazards).

5.9  Cement Job Execution

# 5.9.1  Bulk Plant QA/QC

Accurate cement blends are extremely important to the success of any cement job. Care must be taken to ensure that the blend matches the blend tested in the laboratory.

All cement blends must be blended in accordance with the written procedures established by the Service Company providing the cement blend. In addition, the personnel blending and/or loading the cement must be properly trained and/or certified bulk plant operators.

The cement blenders and all associated equipment should be regularly maintained and inspected to ensure there are no leaking valves or any equipment malfunction that could cause improper additive and/or cement concentrations or contamination. Equipment to be inspected includes check valves, pressure relief valves, interior blender surfaces, additive hoppers, and aeration devices. Appropriate actions, including purging discharge lines and blending systems with neat cement, should be taken to ensure no contamination is possible whenever blends are loaded.

Bulk plant scales should be accurate and in proper working order. These scales should be part of a regularly scheduled calibration program. Copies of the calibration certification should be retained at the bulk plant. A certified calibration technician must perform all calibrations.

Bulk plants should be equipped with proper sampling devices to ensure that multiple samples are taken throughout each blend. The sampling device should be located in an area on the discharge line that ensures that excess moisture cannot enter the sample containers.

# 5.9.2  Cement and Additive Lot Numbers

The Service Company providing the cement and/or cement blend must follow all established documented company procedures to ensure that all received neat cement is within acceptable specifications upon arrival at the bulk plant. In addition, the lot numbers of all additives used must be documented for each cement blend. This information should be contained in the paperwork associated with the particular job for which the blend is loaded. A minimum of two samples of at least one gallon each of neat cement or blend shall be documented, labeled, and retained. One of these samples should be retained at the bulk plant and the other sent to the lab for verification testing (if required). If verification testing is required, testing should be conducted with representative samples of location water.

# 5.9.3  Transportation/Storage

All cement blends must be stored in properly maintained bulk storage tanks. This includes physical inspection of the pads and interior surfaces of the tanks prior to loading/bulk transfer. Tanks should be physically swept out after a cement blend is stored in the tank. This will ensure that no contamination is present in the storage tank(s). In addition, discharge, fill, inspection port, and vent valves must be checked in order to determine that no valves are malfunctioning.

Cement volumes to be loaded must take into account any bulk transfer losses that may occur. This is particularly important in an offshore environment where losses in the bulk tanks of a boat and in transfer to the rig may be significant. Boat tanks must also be physically inspected and, if necessary, swept out prior to loading.

# 5.9.4  Mixing and Pumping

The cement spacer(s) and slurries should be mixed at the planned densities. The spacer and slurry should be batch mixed if possible. If batch mixing is not possible, all attempts should be made to ensure that the slurry density is within the acceptable range. Computer aided density control systems are recommended to ensure that the slurry is mixed at the proper density.

Slurry density fluctuations can have adverse affects on slurry properties including: reduced or extended thickening times, free fluid, retarded compressive strength development, extended slurry transition time, and reduced fluid loss control. Additionally, density fluctuations can result in increased ECD, fracturing of weak formations, and the potential loss of well control.

# 5.9.5  Following the Job Design

The pumping of the cement job should be according to planned rates and volumes. The planned rates should be obtained from the output of a computer simulation.  The computer simulation should determine maximum pumping rates allowable and mud removal efficiency at planned pumping rates. The information entered into the computer simulation should be as accurate as possible. This information should include actual mud, spacer, and slurry rheologies, actual caliper log information (if possible), actual tubular configuration, actual fracture and pore pressures, and actual hardware configuration.  Required hardware configuration data includes actual centralizer data, hanger systems, liner packer data, etc. Having accurate input data ensures that the simulation output will be accurate and feasible. Deviating from planned job design can result in poor zonal isolation due to formation fracturing or mud channeling. Due to this, it is recommended that discussions be held with all pertinent parties prior to a deviation from planned procedure.

# 5.9.6  Pipe Movement

Pipe reciprocation and rotation can assist in effective mud removal. Pipe movement assists in mud removal by altering the flow path of the mud and/cement spacer(s) and slurry. Pipe movement can also help to break the gel strengths of mud that may otherwise be bypassed by the spacer and slurry. Reciprocation must be done slowly to ensure that surge pressures are minimized and losses are not induced due to fracturing of formations.  Computer surge programs can supply the maximum reciprocation rate during a cement job.

Proper equipment must be utilized anytime pipe movement is planned. When reciprocating, ensure that enough treating iron has been utilized from the rig floor to the cementing plug container. Pipe rotation necessitates the use of equipment designed to rotate without creating stress on the plug container and treating iron.

# 5.9.7  Computer Collection of Data

All pertinent job data should be monitored and recorded by computerized data acquisition equipment. The data to be recorded should include density of all fluids pumped, rate at which they were pumped, and surface treating pressure. Pressure should be recorded during the entire displacement, regardless if the rig mud pumps are utilized to displace the plug. If possible, return rates should also be recorded. The recorded job data discussed in this section is necessary as a quality control record and for post job analysis and reporting. It can also supply valuable information in the event that the job can not be pumped as planned.

# 5.9.8  Lost Circulation Contingency Plans

Lost circulation contingency plans should be discussed prior to job execution. These plans can include increasing or decreasing cement volumes, altering cement thickening times, altering fluid loss control of the slurry, and increasing or decreasing slurry pumping and displacement rates. Computer aids such as surge analysis programs and ECD programs can minimize the risk of lost circulation. In addition, there are several float equipment designs that can be utilized to help reduce the risk of fracturing formations while running the casing into the wellbore.

# 5.9.9  Spacers

Cement spacers are one of the most critical components in obtaining successful zonal isolation. Spacers serve several functions including: removing gelled mud ahead of cement slurries, water wetting cement contact surfaces, and providing a barrier between the cement slurry and drilling fluid in the wellbore. Cement spacers should be tested for compatibility with a representative sample of the drilling mud. Improper spacer selection may result in poor mud removal and excessive viscosity at the spacer/mud interface.

Multiple spacers may be required if compatibility does not exist with a single spacer.  The first spacer may aid in mud removal, while the second spacer water wets downhole contact surfaces and provides a barrier between the slurry and drilling mud.

There should be a fluid density hierarchy in which each successive fluid pumped into a wellbore is at least 10% heavier than the fluid preceding it. There may be times when this is not possible or practical, although this is generally preferred.

Turbulent flow spacers are generally recommended if a single spacer system is to precede a cement slurry. Turbulent flow may not be possible due to wellbore eccentricity, enlarged wellbore sections due to washouts, and pumping/displacement rate restrictions due to ECD constraints.

Adequate spacer volume is critical to the success of a primary cement job. Spacer contact time and/or annular fill should be discussed during job planning sessions. The spacer volume should be the greater of 5 minutes contact time (minimum) or 1,000’ of linear fill. Some mud systems require that these values be greatly increased or doubled.

# 5.9.10  Displacement

Displacing of cement slurries should take place at planned rates, unless lost circulation is encountered and the contingency plans initiated. The cement unit should displace all squeeze jobs, liner jobs, stab-in jobs, and plug jobs. The rig mud pumps can be used to displace large casing jobs, though the cement unit should monitor and record the displacement pressures. Displacement rates should also be recorded if possible, and thoroughly documented if recording is not possible.

Over-displacing if the plug does not bump should be discussed prior to job execution. Volumes in excess of 50% of the capacity of the float tract should not be exceeded when pumping additional fluid over calculated displacement volume.

# 5.9.11  Multiple Plugs

Top and bottom wiper plugs are recommended for all casing jobs, with the exception of stab-in casing jobs. On stab-in casing jobs, a drill pipe wiper dart or ball should be dropped behind the cement slurry. The spacer ahead of the cement slurry should be pumped ahead of the bottom wiper plug. This ensures that the cement is uncontaminated in the casing. In addition, double plug containers are recommended where possible. The wiper plug container should have a system indicating the launch of the wiper plug.

Trained personnel should follow established procedures when operating a cement wiper plug container. The top wiper plug should be launched as slowly as practical.  Plugs should never be launched “on the fly”. Careful attention should be paid to when the wiper plug actually leaves, due to the potential for fluid bypass in some plug containers prior to plug launch. It is recommended to launch the top plug with spacer whenever non-aqueous fluids are used to displace.

# 5.9.12  Holding Pressure Inside Casing

The casing floats should be tested after the displacement is complete. Once it has been determined that the floats are holding properly, ensure that no pressure is trapped inside the casing due to closed valves on the cement head. The valves on the cement head must remain open as the fluid inside the casing will expand due to heating and thermal expansion of the displacement fluid. If the valves on the cement head are closed, the casing will expand with the fluid inside. This can create a microannulus, which could result in poor zonal isolation and sustained casinghead pressure. The only time it is permissible to trap pressure inside the cemented casing is in the event that the floats have malfunctioned and are not holding pressure.

5.10  Post Cementing Operations

Maintain Full Hole – In order to maintain maximum overbalance pressure, the fluid level must be maintained in the annulus. In addition to maintaining the overbalance pressure, a full hole will give an earlier warning if the well begins flowing and allows for tracking losses.

WOC – Operations on the well following cementing must be done in such a way that they will not disturb the cement and damage the seal or the set cement.

Any pipe movement to complete hanging the casing and activating seals should be finished before significant gel strength has developed. If done after the cement has developed significant gel strength, the movement could cause a microannulus. There is also danger of initiating flow if the pipe is moved in such as way as to swab the well.

If hanging the casing must be performed after cement strength is developed, as when intentionally increasing or decreasing the landed tension in the casing, this should be performed after the compressive strength at the top of the cement is at least 100 psi.

Likewise, pressure testing casing should be done preferably before significant gel strength has developed. Pressure testing can be done after the cement has set but this can result in microannulus formation or damage to the cement sheath. The pressure should be held on the casing for the shortest length of time required to accomplish the test. The effect of pressure testing will depend on the properties of the cement, the pressure at which the casing is tested (and consequently the amount of enlargement of the casing) and the properties of the formation around the cement. Mechanical stress modeling can assist in determining the best time to conduct the pressure tests.

Normally, a minimum compressive strength of 500 psi is recommended before drilling out the shoe of the cemented casing.

See additional discussion of waiting on cement under Mechanical Barriers.

Top Job – A top cement job (that is, one conducted to fill in the annulus when cement did not reach the desired depth for the top of cement) can be conducted immediately after bumping the top wiper plug or can be done after the cement has set.

Consideration must be given to the probable poor displacement when a top job is performed. Because of this, it is unlikely that a top job will achieve isolation. Every effort should be made to ensure that the primary job circulates cement to surface, when required, and a top job be done only as a last resort.

If done immediately after bumping the wiper plug, formations deeper in the well may be broken down. The formations which might be broken down and the impact on the integrity of the well and the annular seal should be considered when using this method.

If the top job is done after the cement has set, consideration must be given to the method of placement, whether it is to be bullheaded or grouted. Bullheading will result in breaking down a formation. If the cement has set and there is not a channel in the cemented annulus, the formation will break down between the top of the set cement and the shoe of the previous casing. If the top of set cement is above the shoe of the previous casing, this method cannot be used. In such a case, the cement will have to be placed by grouting with a small diameter pipe run into the annulus.

# 6.0  Leak Off Tests

6.1  Introduction

Limit Tests (LT, also known as jug tests or formation integrity tests (FIT)) and Leak-Off Tests (LOT, also known as pressure-integrity tests or pump-in tests (PIT)) are carried out during the drilling phase after a string of casing has been cemented and before a new section of hole is drilled.  In these types of casing shoe tests, the cement at the casing shoe is drilled out and a section of new hole (typically 10-20 ft) is drilled, the BOP is closed around the drill-pipe, and the well is slowly pressured up using mud. Casing shoe tests serve the following purposes:

- To confirm the strength of the cement bond around the casing shoe and to ensure that no flow path exists to formations above the casing shoe or to the previous annulus.  If such a flow path exist, the casing shoe may have to be repaired, e.g. by cement squeeze.

- To investigate the capability of the wellbore to withstand additional pressure below the shoe, such that the well is competent to handle an influx of formation fluid or gas without the formation breaking down.

- To collect data on formation strength and in-situ stress that can be used for geo-mechanical analyses and modeling (e.g., wellbore stability and lost circulation prediction).

Most governmental regulatory organizations maintain criteria regarding verification of casing shoe integrity. The US federal regulations that apply for offshore wells in the Gulf of Mexico are given by the Code of Federal Regulations, 30, Chapter II, §250.54 (7-1-’93 edition). Paragraphs (5) and (6) state:

(5) If there are indications of inadequate cementing (such as lost returns, cement channeling, or mechanical failure of equipment), the lessee shall evaluate the adequacy of the cement (…). If the evaluations indicates inadequate cementing, the lessee shall re-cement (…).

(6) A pressure-integrity test shall be run below the surface casing, the intermediate casing(s), and liner(s) used as intermediate casing(s). The District Supervisor may require a pressure-integrity test to be run at the conductor casing shoe (…).These tests shall be conducted either by testing to formation leak-off or by testing to a predetermined equivalent mud weight as specified in the approved APD. A safe margin, as approved by the District Supervisor, shall be maintained between the mud weight in use and the equivalent mud weight at the casing shoe as determined in the pressure-integrity test. Drilling operations shall be suspended when the safe drilling margin is not maintained (…).

Limit tests are carried out until a pre-determined test pressure is reached, confirming that the formation at the casing shoe can sustain this pressure. The test is characterized by a linear response of downhole pressure vs. volume pumped (or time if the flowrate during the test is constant). These tests are frequently used on production wells in mature fields, where fracture gradients are already well-understood.  The limit test is used to confirm the necessary and available margin to drill the next hole section, without leaving a fracture (= potential point-of-weakness) at the previous casing shoe.

LOT tests are carried out at higher pressures to characterize the phenomenon of “leak-off” into the formation at the casing shoe. Leak-off is characterized by a deviation from linearity on the pressure vs. volume curve.  It is associated with the initiation of a fracture at the casing shoe. Pumping beyond the leak-off point will extend the fracture. A shut-in period is normally maintained after pumping to monitor the behavior of downhole pressure. During an extended leak-off test, backflow of drilling mud after shut-in is monitored in order to characterize in-situ stress values. Both limit and LOT tests may be repeated to verify the results of – or assess the changes induced by – earlier tests, drilling operations, and/or remedial squeeze jobs.

Note that if deviations from a linear response are observed in the pressure vs volume curve during casing shoe testing, the following main factors should be considered: (1) leaks on the surface (e.g., chicksan connections, pump plungers); (2) fluid flow around the casing and cement caused by poor cement bonding and channeling; (3) fluid seepage into formation in the open hole section; (4) trapped air/gas or void space in the mud column that has not been properly filled with mud; (5) non-linear deformation of rock being exposed to mud pressure in the open-hole; (6) fracture initiation and growth in the formation.

6.2  Pressure  Integrity Test Procedures

- Perform casing integrity test per applicable regulations.

- Prepare the operator-specified PIT/LOT form.  Note the anticipated LOT on the form and/or a maximum test pressure if specified.  Use a scale that will best utilize the entire plotting area of the form to help identify the break-over points.  Plot only the portion of the casing integrity test data to fit this scale.

- Drill out casing shoe, clean out rat-hole, and drill 10 ft. of new hole.

- Circulate bottom up to condition mud and stabilize properties, especially density.  Be sure mud is free of air bubbles.

- Pull up into casing a few feet.  General industry practice is to not pump a slug of heavier mud or LCM prior to the test.

- Rig up pumping unit, pump mud through test lines to purge air from the system and pressure test surface lines checking for leaks.  Use same pump that was used for the casing test.

- Close the BOP.

- Pump down the drill pipe at ¼ or ½ barrel per minute (bpm) or other designated rate.  MAINTAIN THE CONSTANT PUMP RATE ASSIGNED.

- During the test, record and plot cumulative volume pumped vs. drill pipe pressure for every ¼ bbl pumped, regardless of the pump rate.

- Permeability in the open hole may cause pressure build-up to be non-linear (i.e., the plot curves and does not follow the trend of the casing test).  If this occurs, stop the test, bleed pressure to zero and retest at a pump rate that is ¼ to ½ bpm higher than the previous test.

- Continue pumping until the predetermined maximum pressure is reached or until leak-off is confirmed.

- If pressure decreases sharply while pumping, stop pumping immediately.  Check surface lines and valves for leaks (make any needed repairs) and attempt to retest.

- If pressure levels off far below required pressure or leak-off, pump additional volume (typically 1–2 bbl) to see if a pressure increase will resume.

- If performing a Limit Test (a.k.a. Formation Integrity Test (FIT) or Jug Test), do not exceed maximum pressure required.

- When leak-off is confirmed, or maximum pressure is reached, stop pumping and close the shut-in valve.

- Observe and record shut-in pressure vs. time.  Check surface lines and valves for leaks during this period.  If any are found, repair and retest.

- Release pressure and record volume of fluid bled back.

6.3  Pressure Integrity Test Guidelines

# Rig Up

- A.	Pump

Use a low-volume high-pressure pump, (e.g., cementing unit).  Do not assume that the mechanical barrel counter on the pump is accurate.  Pump strokes are more reliable volume indicators if the pump is in calibration.  The displacement tanks can also be used to monitor volume if properly marked in ¼-bbl increments.

- B.	Shut-In Valve

Use a shut-in valve between the pump and the pressure gauge.  Use the shut-in valve rather than relying on the pump to prevent back flow.

- C.	Pressure Gauge

- A good quality calibrated pressure gauge is required.  Use an electronic gauge or a mechanical gauge with the following features:  liquid filled, 4” or larger, range 25%-50% greater than maximum anticipated test pressure (the lower the better), 10-50 psi increments (the smaller the better), and isolated from pump vibrations and pressure surges.

- D.	Purge Valve - Optional

- If testing without a top drive, install a purge valve on the pump-in sub and use to purge air from the test lines before initiating test.

- E.	Bleed Valve – Optional

If possible, install a bleed valve on a tee between the pump and the shut-in valve. Open valve during shut-in period to see if the shut-in valve is leaking.

## F.	Pressure Test System Before PIT

# Recorded Test Data

The following items are generally recorded for each PIT:

| - Well Identification | - Test Date |
| --- | --- |
| - Casing Shoe Depth (TVD/MD) | - Test Type (casing/FIT/LOT, etc.) |
| - Hole Depth (TVD/MD) | - Pump Rate |
| - Casing Size | - Predicted Leak-off or Pressure Limit |
| - Hole Size | - Fluid Bled Back |
| - Mud Weight at Surface | - Water Depth (offshore) |
| - Mud Type | - Air Gap (offshore) |
| - Pressure vs. Volume (or Time) |  |

As a minimum, pumping data (pressure vs. volume or time), casing shoe depth and mud weight are necessary for even a rudimentary interpretation of the test.  Other data listed above are also useful in interpretation and comparison of tests.

# Drilling Fluid

Before pulling into the casing shoe for the test, circulate and condition the mud to stabilize properties and clean the hole of all fill.  Circulate and condition for the shoe test.  Insure that the mud weight out equals the mud weight in ( 0.1 ppg).  Different mud types and formulations (e.g., water vs. water base mud vs. non-aqueous mud) can produce different PIT results.  Interpretation of repeat tests is simplified if drilling fluid types and properties are consistent with the first test.

- Pumping Guidelines

Pump rates of ¼ to ½ bpm are typical for initial PITs.  Larger volume tests may require higher pump rates.  Pump rate should be constant throughout the test.  Fluctuating pump rates can complicate test interpretation.

Shut-In Guidelines

- A.	Use the Shut-In Valve

Using this valve rather than the pump will eliminate the possibility of fluid leaking past the pump during shut-in.  As further insurance, monitor the shut-in valve (by means of the bleed valve) and other surface equipment for leaks.

- B.	Monitor Pressure Decline

Monitor the shut in pressure in 1 minute intervals until the pressure levels off or can be reasonably extrapolated to a level-off point.  A 10-15 minute shut-in is usually sufficient.  If the pressure does not level-off after 15 minutes it is worth the time (for interpretation purposes) to wait until it does level off or declines to zero.

# 7.0  Post Cement Job Analysis and Evaluation

## 7.1  Material Inventory

One important aspect of the post job analysis is material inventory after cementing operations are complete. A final inventory of material should be completed and compared to the pre-job inventory as described in the cementing execution section. The material mass balance will ensure that the correct amount of cement and additives were utilized during job execution.

## 7.2  Job Data

To further evaluate the cementing operations, the real time data can confirm fluid volumes, densities and rates in accordance with the initial design. Using computer aided design software, the acquired versus predicted can be compared to obtain pressure matching, equivalent circulating densities and confirm well security. When problems occur during the cementing operations, this information can be useful when investigating job failures.

The information collected from the job should be compared to the predicted job design.  Prior to the cementing operation, a checklist can be prepared from the job design to ensure all requirements have been achieved after execution is complete. The checklist should include all critical job information such as rates, volumes, densities, pressures, etc.

All job data should be collected for a complete analysis and can serve as a reference for future wells drilled in the same or similar areas. Capturing the knowledge and sharing information with other operators could drastically reduce the number of flowing wells after cementing in the Gulf of Mexico.

## 7.3  Cement Evaluation

Formation integrity and cement placement and strength are important parameters to be evaluated before drilling the next hole section. There are two methods commonly used to pressure test the formation below the casing shoe to determine the strength. The two methods are Leak-off Test (LOT) and the Formation Integrity Test (FIT). Each test requires the operator to pump fluids at low rates and small volumes until a change in slope occurs in the pressure versus cumulative volume plot. The pressure recorded at the change in slope is used to calculate fracture initiation pressure and gradient.

When the LOT or the FIT results are inadequate, the operator can perform a cement squeeze or chemical treatments such as formation consolidation or fracture sealant squeezes to enhance the formation’s pressure containment integrity or to seal a leaking cement sheath in the annulus. A repeat LOT or FIT may then confirm the squeeze or treatment results in increasing the interval’s wellbore pressure containment. If the cemented annulus is not leaking, the other option is to drill ahead with the existing wellbore pressure containment limitations of the formation until a more competent formation is reached. For weak formation intervals of a few hundred feet or less, one chemical treatment may then be applied to increase the integrity of the entire weak zone instead of repeat treatments for shorter intervals. When drilling longer intervals, setting a contingency string in a competent formation may be a viable option to achieve the desired LOT/FIT.

A more detailed discussion pertaining to formation integrity test procedures can be found in Section 6.0.

In order to effectively evaluate a cement job, one must ensure the objectives of the job have been achieved. The most obvious is supporting the pipe but other objectives will vary depending on the nature of the cement job. Some of these objectives include isolation to prevent erosion caused by annular flow of fluids, seal off and protect water formations, integrity to support deeper strings and to ensure zonal isolation. Based on the required objectives, multiple techniques are available which include temperature, noise and acoustic and ultrasonic cement logs.

7.4  Cementing Matrix

Add discussion about the cementing matrix and its use.

This matrix is to be used to evaluate the potential impact of elements of the cementing process on its success.  The sheet should be completed by the operator during the planning of the well to highlight areas needing improvement.  At the conclusion of each string on which it is used, the scores for each parameter should be evaluated again and used as a post job evaluation.  The sheet can be printed at each of these stages and placed in the well file.  The scores, both by major category and the total can be compiled in a database and, with evaluation of flow, used for process improvement.

The Cementing Matrix and Instructions are in Table One in Appendix Two.

Appendix One – Reference List

1 Bearden, W.G., and Lane, R.D.; “You Can Engineer Cementing Operations to Eliminate Wasteful WOC Time,” The Oil and Gas Journal, July 3, 1961.

Appendix Two – Cementing Matrix and Instructions

See separate Excel file.