SHSD Operation Environmental Evaluation

Environmental Management System ISO 14001 (HP65805)

| SOEE# SHSD-EMS-12B | Title: Cleaning of Fit Test Respirators | Revision: 2 Date: 08/13/08 |
| --- | --- | --- |
|  |  | Approved by: R. Selvey, IH Manager |
| Operation Description: After each fit test, the mask worn by the subject must be sanitized in preparation for the next user. The masks are disassembled and placed in an aqueous germicidal detergent solution. The product is Georgia Steel “Special Respirator Cleaner Plus”, product number FK300. The cleaner contains Active ingredients of Alkyl (C12, c14, and c18) dimethyl benzyl ammonium chloride (5.63%), Octyl decyl dimethyl ammonium chloride (4.22%), Didecyl dimethyl ammonium chloride (2.85%) and Dioctyl dimethyl ammonium chloride (2.11%). The ratio of mixing the detergent into warm tap water is 1 to 256 (1 once per gallon.) The measuring & dispensing of the detergent is via a pre-measured filling mechanism in the cleaner bottle. The respirators are soaked in the detergent solution and then rinsed in the sink with tap water. The used detergent solution is released into the sink drain and flushed with tap water. |  |  |
| Frequency of Operation: Once or twice weekly, 50 weeks per year. |  |  |
| Environmental Impact: This operation has negligible environmental consequences. The concentration of germicidal detergent is similar to that of consumer over-the-counter antimicrobial hand soaps. No controls are in place or are required. The fit test cleaning procedure is documented in IHG SOP IH72450. |  |  |
| Waste Disposal: The used detergent solution is released into the sink drain and flushed with tap water. It is the equivalent of washing a family’s dinner plates and serving utensils. |  |  |
| Qualitative Environmental Impact Risk Assessment: Of CMS actions Negligible Acceptable Moderate Substantial Intolerable |  |  |
| OSH Job Risk Assessment: SHSD-JRA-12 |  |  |
| Revision History: Rev1: Converted from old IH50900 Attachment 9.1 format. No content change. |  |  |