SHSD Operation Environmental Evaluation

Environmental Management System ISO 14001 (IH50900)

| SOEE# SHSD-EMS-04 | Title: Perchlorate Testing | Revision: 1 Date: 09/19/06 |
| --- | --- | --- |
|  |  | Approved by: R. Selvey, IH Manager |
| Operation Description: 0- 2000 ppm calibration solutions of Perchloric Acid in Ammonium Sulfate solution are made. Field samples with potential Perchlorates (in distilled water) are tested with a specific ion conductivity probe. The meter response for samples is compared to the calibration standards. In each batch of tests, 0.173 ml of Perchloric acid and 26 g of ammonium sulfate are used. |  |  |
| Frequency of Operation: 1-4 times per year |  |  |
| Environmental impact: At end of the test, the ammonium sulfate/perchlorate solutions are sink disposed with copious amounts of water for safety reasons. Disposal of the solutions by dilution in large amounts of water is by far the safest disposal method and has negligible environmental consequences. Once diluted into BNL’s waste water system’s volume, all risk of the reactivity/explosion hazard is eliminated. Policy is documented in IHG SOP IH75200. As per “Liquid Effluent Evaluation Form” dated 08/30/04, if analysis of field samples are conducted in a period when the BNL Sanitary treatment Plant is experiencing elevated levels of Perchlorates, permission for sink release will be suspended and the samples will need to be containerized for waste disposal. For PPE used during testing or analysis, washing the outer surfaces over a sanitary sink is appropriate. The PPE can then be disposed of in the non-hazardous trash. |  |  |
| Waste Disposal: - Ammonium sulfate/perchlorate solutions are sink disposed with copious amounts of water for safety reasons. As per “Liquid Effluent Evaluation Form” dated 08/30/04, if analysis of field samples are conducted in a period when the BNL Sanitary treatment Plant is experiencing elevated levels of Perchlorates, permission for sink release will be suspended and the samples will need to be containerized for waste disposal. - PPE used during testing or analysis, washing the outer surfaces over a sanitary sink is appropriate. The PPE can then be disposed of in the non-hazardous trash. |  |  |
| Qualitative Environmental Impact Risk Assessment: Negligible Acceptable Moderate Substantial Intolerable |  |  |
| OSH Job Risk Assessment: SHSD-JRA-04 |  |  |
| Revision History: Rev1: Converted from old IH50900 Attachment 9.1 format. No content change. |  |  |