|  | Assumptions/Methodology (yellowed items indicate TSG approved Assumptions or methods) | Actions Blue – Load Modeling Subgroup, Red – Modeling Team, Yellow – Transmission Subgroup, Green – Generation Subgroup |
| --- | --- | --- |
| Load Forecast Load Forecast | The WECC 2004 L&R for 2008 will be used for the load forecast. - WECC load forecasts include monthly peak and annual energy for each WECC region. The topology adopted for this planning process is more detailed than the WECC topology. The regions in the material published by WECC are: Northwest Power Pool Area US systems, NWPP Canadian systems, Rocky Mountain, Arizona-New Mexico-Southern Nevada, California, and Mexico. The published WECC data must be divided into the RMATS bubbles. The RMATS bubbles were further divided, i.e. two bubbles instead of one for NW, two bubbles instead of one for PG&E, and consistent with RMATS, multiple additional bubbles for the Rocky Mountain states. Therefore, the WECC load forecast must be disaggregated to accommodate the SSG-WI topology - The WECC 2008 HS2A power flow case will be used to distribute loads to the bus bars. Before this distribution, generation station service should be removed from the power flow base case loads so the load forecast information is consistent with the distributed loads. Also, the power flow case should be modified, as appropriate, to capture incremental transmission additions (now to 2008). Hourly load shapes are a key determinant in modeling transmission congestion. Load shapes must be determined for each bubble (all buses within a bubble will use the same hourly shape). 2002 load shapes from WECC historical hourly load data will be used to shape the loads in each bubble. If conversion to 2002 is too difficult or time consuming, the same shapes will be used as in the RMATS study. The Load Modeling group understands the RMATS study used historical year 2000 hourly shapes Losses in the WECC forecast are grossed up to generation output. Losses will be left in the load forecast for the initial 2008 run. Grid View will calculate losses. A decision on whether or not to remove losses will be made following the initial run. - DSM embedded in the load forecast will be estimated using existing information sources. New DSM is assumed to be included in load forecasts. Interruptible loads will not be modeled - For hydro modeling, the GENESYS model requires annual energy load forecasts. It then constructs hourly loads for the entire year based on such factors as day of week, month and temperature. The conversion in HELM is based on historical load shapes for the Northwest and an historic relationship between load and temperature for each month. The net result is regional hourly demand for 2008 given a particular temperature scenario (from the 1929-78 historical record). The GENESYS model will be used to calculate NW loads and load shapes. - The original plan was to use GENESYS to produce low and high hydro generation cases for the NW, including associated loads. However, due to the difficulty of distributing the simulated hydro generation to the individual plants, recent historical hydro generation will be used: 1999 represents a wet year, 2002 represents an average year and 2003 represents a dry year. The Council will use historical temperatures for these years to adjust the 2008 and 2015 load forecasts to ensure that the hydro and load data are correlated. | - LM will propose and the TSG will approve the load forecast to be used. (COMPLETED) - To disaggregate the WECC load forecast to accommodate the topology (bubbles), CAISO will disaggregate PG&E loads, the Council & BPA will disaggregate NW loads, and RMATS will be used for Rocky Mountain state loads (LM is lead). For other divisions from regions into bubbles, hourly load data (FERC 714) and the RMATS study load divisions will be used to provide ratios to be used for the divisions from regions into areas (bubbles). - The Modeling Team will distribute loads to the bus bar using GridView and the seasonal power flow cases - Load Modeling should remove generator loads from the power flow case (generator loads are netted against plant generation) (COMPLETED) - Doug Smith will coordinate a review of the power flow cases by the Transmission Subgroup to ensure they reflect incremental transmission additions for 2008 – Modeling Team coordinating further review of transmission representation in the 2008 HS2A PF case. - Load Modeling will recommend to the TSG the basis to be used for determining hourly load shapes. (COMPLETED) The shapes use historical data. State energy offices and liaisons for sub-regional studies will be given the opportunity to review proposed hourly shapes before the TSG adopts. - The Modeling Team will calculate losses using Grid View loss modeling and delete losses from the load forecast as directed by the TSG. Load Modeling will estimate the amount of existing conservation and other DSM in the load forecast, using RMATS survey, state energy office and other existing information sources. (This work can be delayed until after other actions are completed for the 2008 study.) - Modeling Team will load the load forecast (as distributed down to the bus level and with hourly shapes) and the associated hydro generation into GridView - Load Modeling will work with the Modeling Team (J. Fazio) to review and agree on NW load results - Mary Johannis – will prepare a write-up for TSG describing the NW load and generation modeling using GENESYS and HELMS - Modeling Team will include the load results from GENESYS in GridView |
| Network Representation | Power flow cases: Only the 2008 HS2A PF case will be used to distribute loads for the 2008 base case. (We may use multiple seasons for the 2015 base case). - Topology: the WECC 22-bubble will be used, with these exceptions: - The single NW bubble will be split into west and east NW bubbles - The single PG&E bubble will be split into three bubbles, to accommodate variations in load types and shapes - The RMATS topology will be used for the Rocky Mountain states, except that the Montana bubbles will be reduced from 2 to 1 With these changes, the SSG-Wi topology will include 33 bubbles. - Control areas: Up to present, the modeling has optimized dispatch and production costs on a west-wide basis, without taking into account rate pancaking and other inefficiencies in system operations. GridView provides some new capability to model dispatch, operating reserves, and wheeling rates by control area, but the model’s approach and algorithm is not fully understood. A high level split of control area modeling will be included if the impact on congestion and production costs is significant | - D. Davies w/ Modeling Team will create a matrix for the TSG that shows whether two power flows or four are necessary (COMPLETED) - WECC (D. Davies) will propose power flow cases to the TSG for decision. Not required for 2008 case. May do this for the 2015 base case) (COMPLETED) - ABB will assist the Modeling Team in moving WECC’s GE-based power flows into GridView. Not required for the 2008 base case. May pursue this for the 2015 case. - Modeling Team will revise topology chart to reflect these changes (J. Austin) - Modeling Team will clarify how GridView models control areas: what data is required, what logic it applies, what results it produces, and issues/impediments it poses. The Team will also test the model’s approach and capabilities with a simple 2 control area split. Results will forwarded to TSG for decision on whether to proceed with control area modeling (assigned to K. Granat) . |
| Transmission Path Ratings & Nomograms | - The Transmission Subgroup started with the WECC path rating catalog, and applied modifications to capture operating limits for a number of paths (see path ratings chart). - Derates to recognize historical OTC limitations will be applied to specified paths. See Notes for the March 29, 2005 TSG meeting for details. Transmission reliability margin may be used to lower the path ratings below TTC - Nomograms take seasonal derates into consideration. Additional nomograms will be collected for possible modeling. Nomograms to be included are listed in Notes for the March 29, 2005 TSG meeting. - Derates to reflect historical operational scheduling limitations, will not be applied in the 2008 study. | - Transmission Subgroup will collect additional nomograms for possible modeling. Marv Landauer will provide a new nomogram for N. of John Day - D. Perry will continue to investigate schedule vs. actuals on key paths, and recommend additional constraints for modeling (COMPLETED) |
| Transmission Forced Outages | - GridView’s ability to model transmission forced outages will not be used in this study. Reason: transmission maintenance outages typically occur during off peak usage only (low impact) and forced transmission outages occur infrequently. (Transmission Subgroup, Sacramento, Feb 3 2005) |  |
| Wheeling rates | - The ABB model allows use of wheeling rates in conjunction with multiple control areas. Wheeling rates must be modeled at the path, not line, level. The rates help to mimic the inefficiencies of multiple control areas and seams - GridView’s capabilities will be tested before deciding to use wheeling rates in the 2008 study. It may be that this modeling will need to be deferred to the 2015 studies. | - Modeling Team will further examine GridView’s requirements and capabilities for modeling wheeling rates. (COMPLETED) A test will be conducted, and the Team will propose an approach to the TSG - Trans SG – Identify the interfaces to represent with wheeling rates and the wheeling rate that should be used. |
| Planning Margin | - A planning margin of about 15 % will be modeled on a sub-regional basis for those areas that are capacity-constrained. For areas that are energy-constrained, resources will be added in the same proportion of installed capacity as is in the current case |  |
| Generation forced and scheduled outages | - Generation forced outages will be modeled probabilistically using GridView’s Monte Carlo capability. The approach used will be repeated between the cases. | - Modeling Team will conduct this modeling in coordination with Gen SG |
| Resources | - Existing resources will be determined on a draft basis by comparing and reconciling SSG-WI 2003, WECC, CEC, and other data bases. The unit level of detail will be collected. - “Sustainable capacity” will be used for existing resources rather than nameplate capacity - Retirements and new resources (present to 2008) will be determined through: - IRP and other resource planning sources - RPS requirements - Target planning margin - Sizing and location of wind resources will be optimized (within IRP parameters) - New DSM will be treated discretely, tracked as a resource but subtracted from load - Resources under 10 MW will not be included. - Initial 2008 studies will be run using the Modeling Team’s initial Unit Commitment template information, coordinated with Tom’s review,, but prior to completion of the SPG review process. - Where unit level of detail is unavailable for thermal resources, assumptions will be made based on type and vintage of plant, cooling technology, etc. | - Gen SG (Tom Carr) has lead to define existing, new and retired generator units for the 2008 case. Tom is coordinating with the states. - Modeling Team will complete the Unit Commitment template using information from existing sources. This will be checked against Tom Carr’s unit definition work. SPG liaisons will coordinate the review of unit representation and unit commitment data for their subregions. - Gen SG, with Michael Milligan (NREL), will determine wind locations, amounts and shapes |
| Criteria for Resource Additions - 2008 | - To be considered for the 2008 study, resources must be under construction –some leeway may be appropriate for new wind resources since they have a shorter lead time - Resources must be in operation by beginning of 2008 unless the resources are critical for resource adequacy and the states are convinced that the resources will be on-line to meet peak needs. | - Liaisons have been appointed to coordinate all resource assumptions with SPGs (D. Perry): - SWAT – Peter Krzykos - STEP – J. Miller - NTAC – M. Landauer - RMATS – Ray Brush & Jim Tucker - CCPG – John Collins - State and provincial energy offices and other experts will be asked to give careful review and comment on all resource assumptions (D. Larson, T. Carr) |
| Unit Commitment | - Thermal unit commitment will be modeled. This is an advance over previous studies - Data requirements for unit commitment includes capacity information, planned and forced outage assumptions, heat rate curves, ramp rates, minimum up/down times, must-run status, start-up costs, non-fuel VOM, and emission rates/constraints information | - Modeling Team has created a template to collect thermal unit data in the level of detail and format required by GridView. Gen SG and the Modeling Team will populate the template as best as possible using 2003 SSG-Wi, SPG, CEC, PNWPCC, and other data sources - Gen SG will coordinate then the draft template with SPGs, state and provincial energy offices via D. Perry and D. Larson - Modeling Team will load the draft template into GridView, and initial studies for the March 29 meeting will be based on this draft information |
| Hydro Modeling | - Recent historical hydro generation that is fairly reflective of the latest Biological Opinion will be used to model hydro generation for the NW - Federal hydro data will be updated using the latest hydrology and projections for 2008 - Low, average and high hydro conditions will be updated and modeled (see load forecast discussion) - The low hydro condition will be selected so that the analysis will also meet the needs of the WECC Resource Adequacy Assessment - A current Corps of Engineers hourly forecast (done for WAPA) of generation will be used for the Central Valley Project in California. - The CEC will provide non-Federal hydro generation for the rest of California. - For Colorado River Projects, peak shaving modeling outside GridView may be used | - J. Fazio will work with the Gen SG to select the historical streamflows to be used for the L, M, and H cases and to provide associated temperatures to allow for load forecast that are correlated with hydro to be used for the Northwest. (COMPLETED) - Gen SG will use ABB’s peak shaving algorithm to model hourly hydro generation for the Colorado River projects and will use the hourly hydro generation provided by WAPA for the Central Valley Project in California. - Gen SG and Modeling Team will work with ABB to integrate GENESYS or other hydro algorithms with GridView (longer-term task) |
| Fuel Prices | - Gas: $5/MMbtu, HH, in 2004 $s; sensitivities @ $6 and $4 - Coal: TBD Default may be the Council’s coal price forecast, to include locational adders for transportation | - Gen SG will coordinate the development of coal price forecast for TSG approval |
| Economic Modeling | 2008 Base case with high and low hydro sensitivities and high and low gas price sensitivities. The modeling will employ the PNWPCC’s hydro modeling capabilities, refined wind generation information, thermal unit commitment logic, and transmission losses | - Modeling Team will create and maintain change log - Modeling for 2008 Base Case: - Verify inputs as intended - Populate model with new inputs and assumptions - Load forecast - Thermal data - New resources - New transmission - Hydro files - Renewables - Fuel Prices - Run 2008 Base Case - Validate Base Case (Modeling Team, Gen SG, Trans. SG, Load Modeling SG) - Rerun with corrections |